STANDARD MICROSYSTEMS CORPORATION

EtherCard PLUS Family LAN Adapters

IBM PC LAN Program Environment

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

1. PC LAN PROGRAM WORKSTATION INSTALLATION

------------------------------------------

1. At the DOS prompt, insert the SMC(or IBM) LAN Support

Program(LSP) Version 1.30 diskette.

2. Type: dxmaid. Press Enter.

3. Press Enter twice.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the option diskette.

6. Install the SuperDisk Drivers diskette in your source drive and

specify the path: A:\NDIS. Press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the hard disk.

7. Press Enter three times.

8. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the EtherCard

PLUS adapter and protocols already selected.

9. Press F4. LSP is installed.

10. Use a text editor to edit the CONFIG.SYS file. Specify at least

20 files in the FILES= line, like this:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

11. Insert the PC LAN Install Diskette in your diskette drive.

12. Type: install. Press Enter.

13. Press Enter three more times.

14. Select the level of function for this workstation: either "Basic

Services" or "Extended Services."  Most of the time, select

the second option.

15. Specify "Workstation."

16. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

17. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

18. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be the same for all

workstations on the network.

19. Follow the prompts to complete the installation until you reach

the Change System Configuration File screen.

20. Usually you should accept the default for the system

configuration file.

The Change System Batch File screen appears.

21. Usually you should accept the default.

2.  PC LAN PROGRAM SERVER INSTALLATION

--------------------------------------

The following section provides instructions for installing PC LAN

on a server.  This procedure uses the LAN Support Program (LSP)

to facilitate the installation process.

1. At the DOS prompt, insert the LSP diskette.

2. Type: dxmaid. Press Enter.

3. Press Enter twice.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the option diskette.

6. Install the SuperDisk Drivers diskette in your source drive

and specify the path: A:\NDIS. Then press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the hard disk.

7. Press Enter three times.

8. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the SMC EtherCard

PLUS adapter and protocols already selected.

9. Press F4. LSP is installed.

10. Use a text editor to edit the CONFIG.SYS file. Specify at

least 20 files in the FILES= line, like this:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

11. Insert the PC LAN Install Diskette in your diskette drive.

12. Type: install. Press Enter.

13. Press Enter three more times.

14. Select the level of function for this workstation: either

"Basic Services" or "Extended Services."  Most of the time,

select the second option.

15. Specify "Server."

16. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

17. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

18. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be used by all workstations

on the network to communicate with the server.

19. Follow the prompts to complete the installation until you

reach the Change System Configuration File screen.

20. Usually you should accept the default for the system

configuration file.

The Change System Batch File screen appears.

21. Usually you should accept the default.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.