PORT FINDER ver. 2.00 by mcTRONic Systems

PORT  FINDER is a very versatile program.  It  will  display

your  computer  type and bios date.  It will detect  and  display

COM1 to COM4 and LPT1 to LPT3 ports that are responding.  You can

swap  COM or LPT ports addresses instead of swapping cables.   If

you are using DOS 3.3 or higher  and have COM3  and/or COM4, PORT

FINDER will  activate  both  COM3  and/or  COM4.   DOS  will then

recognize these ports and you will be able to  use the  DOS  MODE

command.   Any other  programs that  operate  through  DOS should

accept  COM3 and/or  COM4.  Your serial  port count  will also be

increased.  If you have a new IBM PS/2 computer COM3 and/or  COM4

will already be activated.  You can use PORT FINDER to reset both

COM and LPT port addresses in  case you forget   what address  is

what.  PORT  FINDER can also be used in a BATCH  file  to  detect

which COM ports  are present.    An  Errorlevel  is  set  for all

possible  combinations.   An  example  batch file COMMTEST.BAT is

included.

Any  program  that directly addresses the COM or  LPT  ports

will  not be affected by PORT FINDER.  Any program that uses  DOS

to  address  the  COM or LPT ports, will work  with  PORT FINDER,

allowing  you to  swap any  port address you like.  Activation of

COM3 or  COM4 port does require DOS 3.3 or higher  and a  COM3 or

COM4 port.

PORT FINDER  requires  no run time parameters, to display

responding  COM  and  LPT ports  on all  systems,  or to activate

alternate COM ports on PC/XT/AT systems.  DOS 3.3 and higher only

supports  access  to a  maximum  of 4  COM  and  3 LPT ports. The

optional  PORT FINDER  parameters  for parm1  and/or  parm2  with

descriptions are:

d:>PF [parm1] [parm2]

H   - Display this help screen       C23 - Swap COM ports 2 and 3

RC  - Reset COM port addresses       C24 - Swap COM ports 2 and 4

RL  - Reset LPT port addresses       C34 - Swap COM ports 3 and 4

C12 - Swap COM ports 1 and 2         L12 - Swap LPT ports 1 and 2

C13 - Swap COM ports 1 and 3         L13 - Swap LPT ports 1 and 3

C14 - Swap COM ports 1 and 4         L23 - Swap LPT ports 2 and 3

NOTES:

All parameter  letters  can be upper or lower case.  The COM

and  LPT port  addresses displayed   always  indicate  the  final

location after  all  port  swaps  and resets  have been exacuted.

Enter all port numbers with low first then high.

I highly recommend that you not leave any address holes  for

either the COM or LPT ports.  As an example do not have only COM1

and COM3 with no COM2.  Do not have a LPT2 and LPT3 with no LPT1.

If you have one port it should be 1,  two ports should be 1 and 2

and  so forth.  Swap only ports that you have.  While nothing  is

wrong with testing out a port address you do not have by swapping

an  address  you  have to it,  you and  your  system  may  become

confused.   Use  the RC or RL functions to restore  all  standard

port  address  locations.   If you have any  problems  with  your

system after using PORT FINDER then stop using it.

TECHNICAL DATA:

DOS assigns COM and LPT numbers, from LOW to HIGH, to all

responding  I/O  ports  in  order  as  they  are  found  and non-

responding I/O ports addresses are skipped.

MACHINE         COMx             I/O            INT.        MEM

ALL             COM1            3F8 HEX         IRQ4       40:00

ALL             COM2            2F8 HEX         IRQ3       40:02

PC/XT/AT        COM3            3E8 HEX         IRQ4       40:04

PC/XT/AT        COM4            2E8 HEX         IRQ3       40:06

PC/XT/AT        COM5            2E0 HEX         IRQ4       NONE

PC/XT/AT        COM6            3E0 HEX         IRQ3       NONE

PS/2            COM3           3220 HEX         IRQ3       40:04

PS/2            COM4           3228 HEX         IRQ3       40:06

PS/2            COM5           4220 HEX         IRQ3       NONE

PS/2            COM6           4228 HEX         IRQ3       NONE

PS/2            COM7           5220 HEX         IRQ3       NONE

PS/2            COM8           5228 HEX         IRQ3       NONE

SERIAL PORT COUNT IS AT MEM 40:11 BITS 2, 3, AND 4.

ErrorLevel              COM1  COM2  COM3  COM4

0                     0     0     0     0

1                     X     0     0     0

2                     0     X     0     0

3                     X     X     0     0

4                     0     0     X     0

5                     X     0     X     0

6                     0     X     X     0

7                     X     X     X     0

8                     0     0     0     X

9                     X     0     0     X

10                     0     X     0     X

11                     X     X     0     X

12                     0     0     X     X

13                     X     0     X     X

14                     0     X     X     X

15                     X     X     X     X

(X means COMx is there)

PORT ADDRESS     MEM

LPT1   3BC HEX       40:08 (may be included on MDA or other card)

LPT2   378 HEX       40:0A (may be addressed as LPT1 on PC,XT,AT)

LPT3   278 HEX       40:0C (may be addrresed as LPT2 on PC,XT,AT)

DISCLAIMER:

PORT  FINDER is not guaranteed in any way.  Use at your  own

risk!   I have made every attempt to get it right but testing  it

on  every  machine configuration is not possible.   If  you  have

problems with PORT FINDER then stop using it and let me know.

This  program is yours to use and freely copy.  You can  not

sell  PORT FINDER or change any fee for its use.  A  distribution

fee of not more than $5.00 can be charged with my written consent.

Thank You,

James D. McDaniel

Owner/Programmer

mcTRONic Systems

7426 Corwall Bridge Ln.

Houston, TX  77041-1709

(713) 462-7687

*   If  you  need  a good mail list  program  then  send  for  my

shareware program MAILALL.   It is the best and  least exspensive

mail list program you can buy for a small company or personal use.

MAILALL  will maintain the names of up to 20 address  files  each

with  4000 addresses on a hard disk or 1500 addresses per  floppy

disk.   There are nine data fields and you  can sort the  address

file by any two that you wish.  MAILALL will print Labels (up  to

four across), Reports, and Envelopes.  You can edit or print  all

functions  based on address search matches from one to  all  nine

fields.   MAILALL  can import or export WORDSTAR  mailmerge  data

files and can export MULTIMATE ADVANTAGE II mailmerge files.   It

cost  just $27.50 shipped FIRST CLASS MAIL to you.  Send  for  it

today!

PORT FINDER, COPYRIGHT (C) 1988 by mcTRONic Systems

MAILALL, COPYRIGHT (C) 1987, 1988 by mcTRONic Systems

DOCUMENT DATE:  September 7, 1988