Comm_TP4.PAS Ver. 1.50 - RS-232 Support for IBM Compatibles - Documentation

(c) Copyright, 1989

Kevin R. Bulgrien

November,  1989

c/o LeTourneau University

Microcomputer Services

P.O. Box 7001

Longview, TX 75607

INTRODUCTION

This utility is an answer to the many public domain serial port routines which

I have acquired in the quest for good RS-232 support under Turbo Pascal. Unlike

the others, this one works.  Not only does it work, but it also allows you to

use as many COM ports as you want at the same time.  Designed to be relatively

simple and easy to use, I hope many will benefit from it.

This code only works under Turbo Pascal versions 4.0 and 5.x.  For functionally

equivalent routines that work under Turbo Pascal 3.0, look for COMM_TP3.  Also,

a Turbo C version called COMM_TC2 is available.  While it was written for Turbo

C 2.0, it will probably also work for lower versions as well.  Additionally, I

am in in the process of developing a Turbo Assembler version called Comm_TA1.

DISTRIBUTION POLICY

The following files must all be present whenever this package is distributed:

COMM_TP4.PAS  -  Turbo 4.0 & 5.x compatible TPU source code

COMM_TP4.DOC  -  Documentation for the TPU source code

COMM_TTY.PAS  -  Demo terminal emulator which uses COMM_TP4

This package may be used in any non-commercial application without restriction,

although I do require that the source code not be distributed in a modified

form unless you clearly identify the modifications and do not remove any of

my copyright information, distribution policy, or other relevant documentation.

If you wish to use this package for development of software which will be sold

or released with a shareware notice, I do require that a one-time registration

fee of $10.00 be paid to me.  Because this source code is a platform from which

one can build other tools, you are not exempt from this requirement if you have

to modify or add to my code in order to make it fit your specific application.

A letter which acknowledges your payment will be sent to you by return mail.

I specifically prohibit any distribution of the source code where a disk fee is

charged, unless I have granted written permission to the specific vendor making

such a charge.  A general access fee to an electronic bulletin board system is

not considered a disk fee.

I also specifically prohibit any person or entity from claiming copyright on

this material without first obtaining my personal, written consent.  Monetary

compensation to me will be required for any such agreement to be valid.  This

does not infer that you may not copyright material which uses this product.

DISCLAIMER

While I have extensively tested the routines in this package, I will not be

held responsible for any consequence arising from the use thereof.  It is the

responsibility of the user to determine whether or not the enclosed routines

will satisfactorily perform the required functions.

This software directly accesses the Intel 8250 UART as well as the Intel 8259

interrupt controller hardware.  Though they are IBM standards, it is possible

that some manufacturers could use different hardware to supply these functions,

in which case, program compatibility would be questionable.

CREDITS

Credit for this product must go to the various authors who showed me that if I

wanted something that worked, I would have to write it myself.  I won't try to

identify the items, but I obtained at least twelve serial port handlers from

GEnie which could not satisfy my meager requirements.  It amazing to see how

much code is out there which either:  1) does not perform satisfactorily, 2)

has inadequate documentation, 3) is so complex that only an expert could use

it, or, 4) does not contain modifiable source code.

All of the enclosed code is completely original to me, and has not been stolen,

borrowed, or purchased from any other source.

Credit for the information on the IBM compatible hardware must go to the

Prentice-Hall book "Systems Software Tools" by Ted J. Biggerstaff.  I was

amazed to find a book that so clearly outlined the IBM PC interrupt and serial

port hardware.  The world of IBM PC technical information is a desert, and

this book is a rare oasis in the midst of it.

Additionally, credit must also go to the "DOS Programmer's Reference" by Terry

R. Dettmann which is published by the QUE corporation.  I would recommend it

to any programmer who needs a good DOS or BIOS reference.

FEATURES

- Easily expandable interrupt routine

- Interrupt driven send and receive routines

- Carrier Detect monitoring for modem support

- Turbo Pascal 4.0 & 5.x compatible TPU source code

- Convenient TPU format keeps your code uncluttered

- Easy to customize for your hardware, and software requirements

- Uninstalls the interrupt handlers on abnormal program termination

- Number of COM ports limited only by buffer sizes & Turbo memory limits

- A TPU which handles 4 ports can be less than 8K in size

- Easy to use even if you don't understand how it works

- All COM ports handled concurrently in the background

- Example TTY and port setup routines included

- Optional error checking and error messages

- Extensive documentation for software use

- Heavily documented source code

DESIGN PHILOSOPHY

This is a rather grand heading, but I think now is a good time to share the

philosophy used while writing this software.  This, along with a look at the

code and the rest of the documentation, will help you to decide if Comm_TP4

is for you.

Most serial applications which I have been involved with were very simple in

nature.  I did not require a super-high-performance-do-everything-imaginable

piece of software, and as a result, I wrote a port handler which reflected my

particular needs - but also a little bit more.

For example, the simple monitoring of Carrier Detect makes this an ideal

package to write online BBS support programs with.  One of the university

students here has used my package to write a complete online game.

Also, early versions of Comm_TP4 only supported standard DOS ports, but since

I saw that some people would need non-standard setups, I rewrote the code so

that it would allow most any setup.  I have yet to need such a feature, but it

is there and you can be sure that I will keep on adding more support items

like this as time goes on.

Perhaps one of the most important guiding principles I have tried to abide by

can be summed up with a KISS (Keep It Simple Stupid).  I have seen one or two

communications packages that provided a plethora of low level routines which

could be used to manipulate every aspect of the hardware.  I have deliberately

kept this type of code out of Comm_TP4 in the interest of providing a simple-

to-use package rather than an it-does-everything one.  I figure that if someone

knows how to use the low-level operations, they probably know enough to be able

to add them to my code.  Because most people do not need complicated routines,

I felt I would gain more friends by making Comm_TP4 easy-to-use than enemies by

making Comm_TP4 "incomplete".  Make no mistake, though, "incomplete" is a very

relative term, and I really don't believe it applies here.

In summary, I am confident that the routines are high-performance enough to

satisfy the bulk of serial port applications.  They do not, however, provide

a do-everything-imaginable user interface, and yet, there is little that one

cannot do.  Easy-to-use, flexible, and growing are the key words.

GETTING STARTED - DOES IT REALLY WORK?

If you want to prove that these routines really work, first compile the source

file COMM_TP4.PAS into a TPU file with your Turbo compiler.

IMPORTANT NOTE:  Never compile interrupt driven routines with the stack

checking code enabled.  Doing so will cause the program

to crash whenever the interrupt is invoked.

Next, compile and run the COMM_TTY program as it is distributed.  What you will

see is a crude terminal emulation program which uses the COMM_TP4 routines.  It

is capable of receiving data from several COM ports at the same time.  The

screen echoes data from the "logged" COM port and buffers the data from other

ports.  When a different COM port is logged, the buffered data is sent to the

screen.  Also provided is a setup routine that lets you select a baud rate from

110 to 38400.  The other protocol attributes are also selectable.

AN EXAMPLE OF HOW EASY IT IS TO USE COMM_TP4

All it takes is a few simple commands.  For example, let us set up the serial

port to 2400 baud, 8 bits, no parity, and 1 stop bit.  Then, we'll install an

interrupt handler and send a string out the port, and wait for a short

response.  Lastly, we will shut off the interrupt handler.

The program that follows will work with COM1 or COM2 without any modification

to the TPU source code that has been included with this documentation.

PROGRAM TestPort;

USES Comm_TP4;

CONST

Com = 1;  { Use COM1, use 2 for COM2 }

VAR

InputData : CHAR;

BEGIN

SetupCOMPort (Com, ORD(B2400), 8, ORD(None), 1);

InstallInt (Com);

WriteCOM (Com, 'This really is easy.  Isn't it?');

InputData := TimedReadCOM (Com);

RemoveInt (Com);

END.

That is all there is to it.  The Comm_TTY example may be intimidating because

of its size, but that's just because it is a whole terminal emulator.

To be sure, most applications will need a bit more than what is shown here, but

that's what the rest of the documentation addresses.  I just thought it would

be good to emphasize the ease of using Comm_TP4 before delving into the deep,

dark world of technical information.

HOW TO USE THE INTERRUPT HANDLERS IN YOUR PROGRAM

You may invoke SetupCOMPort to change the settings of the COM ports at any

time.  Unless you are sure the port is already set up correctly, you should

use it before you attempt to communicate with the port.

You may turn DTR and RTS on and off with Set_DTR_RTS in order to provide

simple hardware handshake capability.

You may install COM port handlers with InstallInt at any time and any order.

You may uninstall COM port handlers with RemoveInt at any time and any order.

You may force any of the input or output buffers empty at any time by using

Procedure EmptyBuffer.  This allows you to abort any interrupt driven transmit

routine.  It also allows you to flush unwanted input from the receive buffers.

To read data that has come in from a port, one of the following is required:

1) FUNCTION ReadCOM (Com : BYTE) : CHAR;

FUNCTION TimedReadCOM (Com : BYTE; VAR Data : CHAR) : BOOLEAN;

where Com <= MaxPorts.  Usually Com will refer to the DOS COM port

number, but it does not have to.  In any case, the global value

COMNmbr [Com] indicates the DOS COM port number.

ReadCOM will wait until data becomes available if there is no data in

the receive buffer - forever if necessary.

TimedReadCOM will abort and return FALSE if data does not become

available in a relatively short period of time.  The data is placed in

the VARiable parameter 'Data' when TimedReadCOM returns TRUE.

2) DisableInts;

CharReady := InTail [Com] <> InHead [Com];

EnableInts;

where CharReady is a user defined BOOLEAN that will be TRUE if data is

waiting in the buffer or FALSE if not data is waiting.

DisableInts;

CharData := CHR(InBuffer [Com, InHead [Com]]);

InHead [Com] := (InHead [Com] + 1) MOD (MaxInSize + 1);

EnableInts;

where CharData is a user defined CHAR variable that will contain the

next item in the buffer IF AND ONLY IF CharReady is TRUE.  The item is

removed from the buffer with the statement following the CharData

assignment.

To write data to a port, use one of the following methods:

1) FUNCTION WriteCOM (Com : BYTE; Data : STRING) : BOOLEAN;

PROCEDURE WriteCOM (Com : BYTE; Data : STRING);

PROCEDURE IWriteCOM (Com : BYTE; Data : STRING);

where Com <= MaxPorts.  Usually Com will refer to the DOS COM port

number, but it does not have to.  In any case, the global value

COMNmbr [Com] indicates the DOS port number.  Data is either STRING

or CHAR information to be sent to the port.

Both the function and procedure declarations of WriteCOM will wait for

all of the data to be sent.  If it has trouble sending the information,

both will time out and return.  Use the function declaration if you

want to be notified when this happens.  A returned FALSE indicates that

the transmission failed.

IWriteCOM returns immediately after placing all data in the transmit

buffer.  The data is sent in the background while your program does

other things.  It is not practical to use IWriteCOM for sending single

characters since it always calls WriteCOM once.  No provision is made

to handle transmission failures.

2) PortReady := ((PORT [COMPort [Com]+LSR] AND $20) = $20);

where PortReady is a user defined BOOLEAN variable which will be TRUE

if the port Transmitter Holding Register is ready for a character to

send to the port.

PortReady := PortReady AND CTS [Com] AND DSR [Com];

This statement allows you to see if the CTS and DTR lines indicate that

the other serial device is ready to receive data.  It may be omitted if

you do not want to check these lines.

PORT [COMPort [Com]+LCR] := PORT [COMPort [Com]+LCR] AND $7F;

PORT [COMPort [Com]+THR] := ORD (CharData);

This statement places the data in the transmit buffer.  CharData is a

user-defined CHAR variable that contains a character to send to the

port.  PortReady, as defined above, MUST be TRUE before placing this

data in the Transmitter Holding Register otherwise the previous data

item will be lost.

NOTE:   These low-level code fragments are provided so that you can write

receiver and transmitter routines which support your particular needs.

See the following technical information for more details.

SETTING COMM_TP4 UP TO USE IN YOUR OWN PROGRAMS - TECHNICAL INFORMATION

In order to use the serial routines, simply include Comm_TP4 in your USES

statement at the top of the program.  Of course, you can add or delete routines

from the TPU source code to make it fit your applications.  Because of Turbo's

smart linking, however, it is not necessary to remove code that you do not need

in a particular program since unused code is not added to the final .EXE file.

All of the variables necessary for developing your own customized input/output

routines are in the INTERFACE section so that you do not have to recompile the

unit if you include new serial I/O source code in your program.

Several conditional compilation directives at the top of the source file allow

you to compile TPUs customized for special applications:

{$DEFINE ErrorChecking} - Inclusion of this directive will enable various

error checking code that could help you debug a

program under development.  It can also be used

as a basis for your own error checking routines.

To safeguard against fatal run-time errors, the

procedures InstallInt & RemoveInt always contain

some error checking whether this directive is

present or not.

{$DEFINE NoMessageCode} - This directive is used to eliminate all code

which would send error messages to the screen.

It does not disable error handling.

The global variables ErrMsgX, ErrMsgY, and

ShowMessages are also disabled with this

directive.

Error messages consist of a beep, the procedure

or function name, the handler number, and the

error description.

{$DEFINE FWriteCOM}     - WriteCOM is the main procedure which is used to

send data to a port.  It times out if the data

cannot be transmitted for some reason.  Placing

this directive at the top of the source file

causes WriteCOM to be made a function that

returns a boolean value of TRUE if all of the

data was transmitted successfully.

I would suggest that you use the {$UNDEF} directive in place of {$DEFINE} when

you do not want to use one of these compilation directives.  By doing so, you

will always realize that the directive exists when you casually look over the

source code.

Several CONSTants exist which can be used to customize Comm_TP4 for a variety

of purposes:

MaxPorts   - The maximum number of ports which can be supported at the

same time.  The maximum possible setting of MaxPorts is

restricted only by Turbo Pascal's space limitations for

TPU files.

MaxInSize  - The maximum size of the input and output buffers.  Both are

MaxOutSize   provided so that memory can be conserved if the sizes needed

for input and output are different, and both affect the

maximum possible value of MaxPorts.

COMNmbr    - A predefined array which can be used to identify the actual

COM port serviced in case the order does not agree with the

internal port/buffer numbers.

Actual COM number = COMNmbr [Software Port Number]

COMPort    - A predefined array of base addresses for the INTEL 8250

serial ports available.  Comm_TP4 comes preconfigured with

the standard IBM/DOS addresses used for COM1 through COM4 but

can be set up for any valid port address.

To access the registers of a given port, use the following

constants:

THR - Transmit Holding Register

RHR - Receive Holding Register

DLL - Divisor Latch Register Least Significant Byte

IER - Interrupt Enable Register

DLM - Divisor Latch Register Most Significant Byte

IIR - Interrupt ID Register

LCR - Line Control Register

MCR - Modem Control Register

LSR - Line Status Register

MSR - Modem Status Register

For example, to access the Modem Control Register, use the

following expression where Com is the port handler number:

PORT [COMPort [Com] + MSR]

One exception exists however.  The THR, RHR, and DLL occupy

the same address, as do the DLM and IER.  To distinguish

which registers you are accessing, use the LCR register.

Do not be confused by THR and RHR, as they are really the

same register, but functions differently depending on whether

you write to or read from it.  In any case, to select the

THR, RHR, and the IER, use the following commands:

PORT [COMPort [Com]+LCR] := PORT [COMPort [Com]+LCR] AND $7F;

To select the DLL and DLM registers, use:

PORT [COMPort [Com]+LCR] := PORT [COMPort [Com]+LCR] OR $80;

IRQNmbr    - A predefined array which identifies the interrupt priority to

be used for each port.  Lower IRQ numbers cause the port to

receive attention first if a conflict arises.  This number

depends on a hardware setting, and can have fatal effects

if numbers other than 3 or 4 are used.  You must know exactly

what you are doing if you change these numbers.  Comm_TP4

comes preconfigured with the standard IBM/DOS IRQ numbers for

COM1 through COM4.

ChainInt   - A predefined array which is not used at this time.  It is

here to remind me to make an enhancement that will allow a

Comm_TP4 interrupt handler to be placed in series with a

pre-existing interrupt handler.  For example, many memory

resident programs place their own keyboard interrupt handler

in series with the original DOS interrupt handler so that

they can invoke themselves with unique key combinations.

The possibilities are intriguing...

NOTE:  All of these CONSTants could be changed to VARiables so that your

program could prompt the user for the correct information about his

machine.  Dynamically allocated heap variables would come in handy if

you wanted to change the MaxPorts, MaxInSize, and MaxOutSize constants

at run-time.  Perhaps Comm_TP4 will incorporate these ideas later on.

Several VARiables exist which can be used to poll Comm_TP4 for the current port

status.  They can also be used to build your own low-level Comm_TP4 functions

for a variety of purposes:

OutBuffer    - These two dimensional character arrays are the output and

InBuffer       input buffers for all of the ports which can be serviced

by the Comm_TP4 routines.  CONSTants MaxPorts, MaxInSize,

and MaxOutSize directly affect the size of these arrays,

so you should optimize their settings for your particular

application.  All buffer operations are outlined below.

OutHead      - These one dimensional word arrays are the pointers into the

OutTail        buffer arrays.  They are used so that the buffers become

InHead         circular queues.  The head points to the next item in the

InTail         buffer, unless (Head = Tail) in which case the buffer is

empty.  The tail points points to the place where new data

will be placed when it arrives, unless (Tail + 1) MOD

MaxSize = Head in which case, the buffer is full.

When you want to take something out of a buffer:

1) You must first be sure the buffer is not empty.

2) Get the character with Buffer [Com] [Head]

3) Adjust the head pointer to (Head + 1) MOD MaxSize

When you put something into the buffer:

1) You must first be sure the buffer is not full

2) Put the character into Buffer [Com] [Tail]

3) Adjust the tail pointer to (Tail + 1) MOD MaxSize

In the above examples, the "In" and "Out" qualifiers have

been left out.  Also, "Com" refers to the handler number.

NOTE:  ALWAYS disable the hardware interrupts before you

do anything with the input or output buffers and pointers.

Not doing so will cause hard to find run-time errors, but

NEVER forget to re-enable the interrupts after you disable

them.  Not doing so will disable other crucial hardware

operations along with your serial port I/O functions.

IntInstalled - This one dimensional boolean array indicates when a port

handler has been installed.  You should never change its

value as doing so will eventually cause the computer to

crash since it controls the modification of the system

interrupt vectors.  To read the install status, check the

following expression, where Com refers to the appropriate

handler number:

IntInstalled [Com]

ErrorCode    - ErrorCode is set to 0 after a serial port operation has

ErrorPort      occurred without any errors.  If it is not zero, ErrorPort

indicates which port the error occurred on.  ErrorCode will

be one of the following:

1 - Invalid port number

2 - Port already installed

3 - Port not installed yet

4 - Timeout writing to port

5 - Timeout reading a port

ErrorCode and ErrorPort remain valid until the next serial

port operation is requested.

ShowMessages - This global boolean variable is used to tell Comm_TP4's

error handler whether or not it should display messages

on the screen.  TRUE allows messages to be displayed, and

the default value may be set in the unit initialization

code.  If the {$DEFINE NoMessageCode} directive is used,

this variable will not exist.

ErrMsgX      - Like ShowMessages, these global variables are used by the

ErrMsgY        Comm_TP4 error handler.  The {$DEFINE NoMessageCode}

directive also removes them from the code.

ErrMsgX and ErrMsgY determine where the messages will be

printed.  A coordinate of 0 will cause the message to print

at the current cursor coordinate given by WHEREX or WHEREY.

Their default values are 0.  If you want the messages to be

written at a specific screen position, you must set them to

the desired coordinates before you request a serial port

operation which uses error checking.  The default settings

may be specified in the unit initialization code.

DTR_RTS      - This array controls whether or not the Comm_TP4 routines

are allowed to change the status of DTR and RTS.  Programs

which use modems may not want the software to drop DTR and

RTS since this can cause the modem to hang up.  A TRUE

enables DTR and RTS setting, and the default value may be

specified in the unit initialization code.

Be wary of hardware handshaking...  I have found that my

modem refuses to work when I set DTR_RTS TRUE even though

it theoretically should work.  Actually, I prefer to write

my applications so they turn DTR and RTS on, them leave

them on.  Hardware handshakes end up being headaches more

often than not.

CD           - These one dimensional boolean arrays indicate the current

CTS            status of each port's input lines.  They are correctly set

DSR            when the program starts up, but only track changes in

RI             status while an interrupt handler is installed AND while

the Modem Status Change interrupt type is enabled.  When

you install an interrupt handler, each array is set again.

The current status of an individual line can be read while

an interrupt handler is not installed as well.  To do so,

use one of the following statements, where Com refers to

the port handler number:

CD  [Com] := ($80 AND PORT [COMPort [Com]+MSR] <> 0);

CTS [Com] := ($10 AND PORT [COMPort [Com]+MSR] <> 0);

DSR [Com] := ($20 AND PORT [COMPort [Com]+MSR] <> 0);

RI  [Com] := ($40 AND PORT [COMPort [Com]+MSR] <> 0);

Framing      - These arrays are used to keep track of the number of

Break          communications errors which occur on each port.  This is

Overrun        perhaps a trivial operation, however, it does serve to

Parity         document how to detect the error conditions.

Each array element is initialized to zero on startup, then

they are incremented every time a corresponding line status

error interrupt occurs.  You may change their values at any

time without affecting the operation of the software.

Several VARiables exist which are deliberately not included in the INTERFACE

section of the TPU.  They are critical to the internal operation of Comm_TP4,

and should NEVER be altered by external routines.  They are documented here

for completeness.

OldIntVector - This array keeps track of the interrupt vector which should

be restored when Comm_TP4 uses RemoveInt to uninstall an

interrupt handler.  InstallInt initializes OldIntVector

as necessary.

ExitSave     - A pointer which is necessary for linking into Turbo's exit

procedure structure.  This is what allows the automatic

uninstallation of Comm_TP4 interrupts whenever the program

terminates - whether normally or abnormally.  ExitSave is

initialized on program startup.

IRQMask      - A bit map of all IRQ levels which currently have Comm_TP4

interrupts installed.   This makes it fairly easy for the

InstallInt and RemoveInt procedures to know whether or not

they need to alter the interrupt vectors and the 8259 IMR.

The primary reason for using IRQMask, however, is to allow

the interrupt handler to enable higher priority interrupts

while preventing Comm_TP4 from interrupting itself.  Only

InstallInt and RemoveInt may modify the value of IRQMask

after it is initialized to zero on program startup.

KNOWN PROBLEMS WITH COMM_TP4

At this time, the interrupt handler is not efficient enough to operate well

at high baud rates on slow PC's if you have too many of the interrupt types

enabled.  It will fail, for example, if you try to communicate at 9600 baud

on a 4.77 MHz machine while the Line Status, Modem Status, and Receive

interrupt are all activated.  I plan to explore the nature of the problem

further in the near future.

CONSIDERATIONS FOR PROGRAMS WHICH USE INTERRUPT HANDLERS

Interrupt handlers operate in ways that can confuse those who are not used to

working with them, so, the following discussion may be helpful if you are

planning to modify Comm_TP4.

Keep in mind that an interrupt can occur at ANY time.  This means that a Pascal

statement could be interrupted during its evaluation.  Usually this is not a

problem, but sometimes it is very much a problem.  This is the case when both

the interrupted code and the interrupt handler need to access and/or modify the

same data.  Consider the following trivial, unrealistic, but enlightening code

fragment:

X := 1;

{ Interrupt1 }

Y := 2;

Z := X * Y;

IF (Y = { Interrupt2 } Z)

THEN WRITE ('EQUAL')

ELSE WRITE ('NOT EQUAL');

It seems obvious that the else clause will never be executed, but this can be

very untrue in the case of a program which uses interrupts!  If, for example,

an interrupt handler interrupted this program at the comment { Interrupt1 },

we can easily see that the program would not do what we expect it to do if the

interrupt handler changed the value of X.  A less obvious flaw shows up though,

if you consider an interrupt occurring at { Interrupt2 }.  Here, both Y and Z

could actually equal 2, but since the interrupt changes Z before it finishes

comparing the values, the comparison fails.  Once again, this is a non-sensible

example, but try to imagine the same thing happening with our serial port

handler buffers and buffer pointers...

The whole point is that certain operations should not be interrupted, and this

is where the DisableInts and EnableInts inline procedures come into play.  They

are used to temporarily suspend interrupt processing and to re-enable interrupt

processing when the program has passed the danger zones.  This is the reason

you will see them sprinkled throughout the Comm_TP4 routines, and if you plan

to write you own serial I/O routines, you will need to know where to put them.

1) Always disable interrupts while setting the 8259 interrupt hardware.

2) Consider disabling interrupts when you are accessing data which could

be altered by the interrupt handler.  The key is to be able to discern

whether or not the effects of an interrupt would cause the program to

operate abnormally -or- if an interrupt could even occur in crucial

areas.  Obviously, if an interrupt has not been installed, it is

pointless to disable it during that section of code.

3) Remember that you must not leave interrupts off for significant periods

of time.  The clock, keyboard, disk drives, etc. all work off of the

hardware interrupt system.  Things could really get flaky if you forgot

to reenable interrupts after turning them off - besides, you would

defeat the whole purpose behind using an interrupt driven routine.

4) Be careful that you don't paint yourself into a corner by disabling

interrupts during an operation that requires the interrupts to be on.

For example, if we disabled interrupts while checking to see if data

was in the input buffer, we had better turn them back on sometime before

the loop returns to check the buffer again, or else you've just created

an endless loop.

Interrupt handlers can be tricky to use, but with the right mix of caution,

bravery, and forethought the results can be impressive.  Don't let the words

of warning scare you off!

SUPPORT

User support may be obtained by contacting me via the LeTourneau University BBS

at (214) 237-2742.  The BBS runs 24 hours a day and accepts calls at 300, 1200,

and 2400 baud.  Since I am the SysOp of this board, this is the fastest way to

reach me.  My GEnie mail address is K.BULGRIEN, but money being at a premium,

I do not always check in on a regular basis.  I may also be reached during

business hours at (214) 753-0231 ext. 352.

Support will be limited to clarification of the routines I wrote, but don't

holler for help if you haven't bothered to read the documentation first.  I

will be glad to fix any bugs that I may have missed, but I cannot be expected

to debug your routines or enhance my routines for your specific application.

Money does have a way of changing people's minds though...

UPDATES

I will place updates of the enclosed routines on LeTourneau University BBS at

(214) 237-2742.  The BBS runs 24 hours a day and accepts calls at 300, 1200,

and 2400 baud.

I will also place updates in the BORLAND RT on the GEnie information service.

Possible updates may include:

1) Run-time allocation/configuration of port handler variables and setup

2) Addition of file transfer protocols

3) Miscellaneous enhancements

4) Other language versions

5) Your ideas...

6) Bug fixes

VERSION HISTORY

Version numbers will always be in the form [a.bc] where [c] indicates a bug fix

or a minor file change, [b] indicates a minor enhancement or rewrite, and [a]

indicates a major addition or rewrite.  I will always increment the version

number when I release a set of files with code changes as I want to maintain

strict control of the status of files which are being distributed.

Beta  ??/87  It was a long time in the making.  The early implementations were

buggy and undocumented since I was just learning the technology.

1.00  11/88  The first officially released version which was placed in the

Borland Roundtable on GEnie.  The code was written under Turbo

Pascal Version 3.01A and I ported the routines to Version 4.0 &

5.0, and then to Turbo C 2.0.  Subsequent versions have been

developed under Turbo Pascal 4.0/5.x and ported to the other

language implementations.

1.01  02/89  Bug fix in Procedure RemoveInt.  It correctly uninstalled the

interrupt code except that it would disable all IRQ interrupts

except the one used by the specified COM port.  (An AND instead

of an OR in the statement writing to PORT [$21].  Uploaded the

new version to GEnie to replace the old one.

1.10, 02/89  Interrupt transmitter added. (Procedure IWriteCOM)

1.20, 02/89  Major improvements corrected limitations and potential flaws

in the code.  Unlimited port support was made possible by

allowing the user to specify IRQ numbers with port numbers and

addresses instead of using the assumed DOS standards.  To use

the additional features, the interrupt handler now processes

all ports with the same IRQ when it is invoked.  Planning for

chained interrupts was started with this revision.

1.30, 03/89  Converted code to a unit file with a separate TTY test program.

Repositioned DataBits parameter in SetupCOMPort so the order

is more intuitive.

1.31, 10/89  Added the conditional compilation directive Online to optionally

disable RTS & DTR from being shut off if the program expects a

modem online at startup.  Added the conditional compilation

directive Debug to optionally disable debugging code.

1.40, 10/89  Added the conditional compilation directive FWriteCOM that lets

you easily convert procedure WriteCOM to a function that returns

a boolean TRUE if the transmission was successful.  Also added

CONST COMNmbr that shows which COM port each handler refers to

in case non-sequential port numbers are used.  Renamed IRQNum

to IRQNmbr to match COMNmbr.  I moved more declarations to the

INTERFACE section so they are visible to the calling program.

Added procedure EmptyBuffer.  Expanded the initialization code

so that it sets up all variables which might be needed by the

calling program.  Renamed the conditional compilation directive

Debug to ErrorChecking.  Rewrote the debugging code and error

message handling by adding in procedure MakeError and function

ValidPort.  Added variables ErrMsgX, ErrMsgY, ErrorCode, and

ErrorPort to make the error handling code more versatile.  Added

the conditional compilation directive ShowMessages to enable or

disable the error messages.  Removed the buffer initialization

commands from InstallInt.  Fixed IWriteCOM bugs:  1) Eliminated

the possibility of it locking up on a full output buffer, and 2)

added a check to see if the interrupt was already on before

attempting to turn it on.  This also improved the through-put

efficiency for the buffers during heavy data transfer.

1.50, 11/89  Removed the conditional compile directives Online & ShowMessages.

ShowMessages is now a global boolean variable which may be set by

the calling code.  Added the compiler directive NoMessageCode

which is used to optionally eliminate the error message support

and the control variables ShowMessages, ErrMsgX, and ErrMsgY.

The global boolean DTR_RTS was added to allow the calling code to

decide if DTR and RTS should be modified.  This replaces the less

flexible conditional compilation directive Online.  Added the

procedure Set_DTR_RTS which is called when DTR and RTS are to be

set.  Fixed a bug in InstallInt and RemoveInt so that they now

work correctly when more than one port handler is installed on a

single IRQ.  Created the header file Comm_TC2.H for the Turbo C

version.  Renamed Carrier to CD and added CTS, DSR, and RI modem

status arrays with the code to track their current states.  Added

the Break, Framing, Overrun, and Parity arrays along with code to

track the number of communications errors.  Streamlined various

parts of the code by removing some unnecessary DisableInts and

EnableInts statements.  Reworked some of the I/O routines.  Fixed

IntHandler so that it safely enables higher priority interrupts

while making sure Comm_TP4 will never interrupt itself.  Added

IRQMask and modified InstallInt and RemoveInt for this feature.

THE END