# CHAPTER 10	VOYETRA SOUND FACTORY§§

This chapter is an excerpt from the documentation for Voyetra's Sound Factory sound development toolkit.  Since Sound Factory supports every major PC sound card, this documentation may also refer to API versions for other hardware platforms.  However, this kit only includes the API versions for the Aria Multimedia System.

PLEASE NOTE:  
The Voyetra Sound Factory API and related utilities included with this development kit are for evaluation purposes only.  If you wish 
to include any portion of the software with your application, a distribution license must first be obtained from Voyetra.

  Contact Voyetra Technologies at:
(914) 738-4500    9 AM - 5PM E.S.T.

## 1.10	DOS AND MULTIMEDIA

While Microsoft Windows with Multimedia Extensions (MME) and Windows 3.1 provide a uniform, device-independent way of adding sound to Windows programs, Microsoft has not provided a way to add device-independent sound to DOS programs.  As a result, developers who wish to add sound to their DOS applications are often faced with the complex task of writing MIDI and digital audio file players and device-dependent interface code for every PC sound card they intend to support.

To address this need, Voyetra Technologies has created a powerful DOS Application Program Interface (API) called Sound Factory, which offers DOS programs virtually the same sound functionality as Multimedia Windows.  Sound Factory is built around the high-level Voyetra Multimedia Player (VMP).

With VMP, an application can simply say "play this MIDI song" or "play this digital audio file" and then get on with another task  while VMP does its job in the background.  Without VMP, the application has to juggle the audio and MIDI playback while trying to keep up with the job it was designed to do.

For example, a presentation program would typically use VMP to play MIDI music and digital audio narrations in the background while it handles the foreground task of presenting an animated screen show.  As the application works with graphics files that appear on the screen, VMP would be transferring digital audio from the hard drive, to its RAM buffer, to the audio hardware.  All the while, it's queuing up and sending a MIDI song file to the MIDI port at precise intervals in order to maintain accurate song tempo.

## 1.20	SOUND FACTORY APIs vs DIRECT CONTROL OF THE ARIA SYSTEM

Although the Aria system can be controlled by writing to the hardware registers, using the Voyetra Sound Factory APIs offers the following benefits for software developers:

·	The API's allow software to run on all Sound Factory-compatible PC sound cards.

·	The APIs relieve the application from having to interface directly with the DMA and interrupt controllers, so that the application does not have to decipher the interrupts generated by the card.

·	The MIDI API (VAPI) provides system timer functions that use 'traps' to call back the application. This guarantees that call-backs will not be re-entered while interrupts are enabled.

·	The digital audio API (AAPI) double-buffers the PCM data and supports 4 different industry-standard data formats. The Digital Audio API functions use simple handshaking and will stop automatically when there is no more data to be played or an overrun occurs during record.

·	The synthesis API for the Aria system (SAPIARIA) provides the ability to load alternative sound palettes by simply specifying the desired sound palette file at load time.

## 2.30	SOUND FACTORY ARCHITECTURE

Sound Factory, like Multimedia Windows, is structured on two levels:

·	The Voyetra Multimedia Player (VMP) is a memory-resident API that is functionally analogous to the MME's MCI layer in that it provides a high level interface for playing digital audio and MIDI files as a background task. Simply put, the VMP is a MIDI and Digital Audio playback engine.

·	A set of low-level APIs (VAPI for driving the MIDI port, SAPI for driving internal synthesizers and AAPI for driving the digital audio hardware) provide device independence for the VMP, as well as offering a rich set of function calls for programs that need direct control over the sound hardware.  For certain sound hardware platforms, a combined version of these low-level APIs, called TAPI, is also available.

µ §

Figure 4.1  OVERVIEW OF SOUND FACTORY API ARCHITECTURE

## 2.40	THE VOYETRA MULTIMEDIA PLAYER (VMP)

The main component of the Sound Factory API's is the Voyetra Multimedia Player (VMP);  an Applications Programming Interface (API) for the DOS environment that provides function calls for real-time control of the PC sound hardware.  For most applications (background music, interactive training, voice annotation, etc.) VMP will contain all the sound-related functions you'll need to control the Aria Multimedia System.

The VMP can best be described as a "playback engine" for MIDI and digital audio files.  By allowing developers to sidestep the complex task of implementing code-intensive record/playback routines, the VMP makes it easy to integrate music and sound into any DOS application.

VMP provides support for:

·	Playing back MIDI song files through external MIDI devices, PC-based synthesis cards or both simultaneously.

·	Recording and playing sound that is captured to disk via audio digitization.

·	Controlling audio playback from CD-ROM drives using Microsoft's CD-ROM Extensions (MSCDEX).

·	Software control of the on-board audio mixer found on many multi-function sound cards.

·	Reading incoming SMPTE timecode via an internal SMPTE reader/generator such as the Voyetra V-24s MIDI/SMPTE interface.

·	Fine event timing through callback functions.

VMP and its underlying control API's are memory-resident programs that are invoked by a software interrupt.

Conceptually, VMP can be broken down into several "parts":  a MIDI File Player, a Digital Audio Player/Recorder, CD Audio Control, Mixer Control and SMPTE Timecode Reader.

### 2.4.1	MIDI File Player

The application is responsible for loading the MIDI file from disk to RAM. When file playback is started, VMP returns control to the application and plays the MIDI file in the background.

VMP can play the MIDI song file using MIDI hardware, a PC-based synthesizer card, or both simultaneously.  It's default playback mode emulates the routing scheme used by Multimedia Windows, thus providing full compatibility with files adhering to Microsoft's Authoring Guidelines for MIDI Files and the Multimedia PC (MPC) spec.

MIDI song files, such as those in the Voyetra MusiClipsä song library, can be created and edited using a MIDI sequencing program, such as Voyetra's Sequencer Plusä Series.

### 2.4.2	Digital Audio Player

The Digital Audio Player plays digitized audio files from hard disk, freeing the application from having to deal with file formats, disk access and DOS re-entrancy.  When file playback is started, VMP returns control to the application and plays the digital audio as a background task.

Additionally, a "record" function transfers digital audio from the digitizing hardware to the disk.  As with playback, VMP returns control to the application.  Thus, digital audio can be recorded while the application is running, making it possible to "narrate" an application in real time.

VMP supports digital audio files in Sound Blaster .VOC, Microsoft .WAV, Voyetra .VOY and raw data formats.

### 2.4.3	CD Audio Control

VMP provides access to PC-based CD audio through the Microsoft CD-ROM Extensions (MSCDEX) calls, also known as "Red Book" digital audio.

Virtually all CD-ROM drives on the market come bundled with MSCDEX and the necessary device driver.

### 2.4.4	Mixer Control

Sound cards that comply with the Multimedia PC (MPC) specification are required to have a software-controllable mixer to control the levels of the various sound sources.

VMP includes a number of functions that provide complete control over the output level and record level of the various sound sources (synthesizer, digital audio, auxiliary input, etc.), master output level, and other mixer parameters.

### 2.4.5	SMPTE Timecode Reader

VMP includes functions to read incoming SMPTE timecode, allowing an application to "know" the current SMPTE time.  Thus, MIDI music, digital audio and PC video screen images may be cued to external sound and video tape recorders via SMPTE timecode.  The SMPTE timecode enters the PC via an internal SMPTE reader/generator such as the Voyetra V-24s SMPTE/MIDI interface.

### 2.4.6	Timer Services

VMP also provides timer services accessible to the application as a high-resolution clock operating at 1 ms resolution.

## 2.50	SOUND FACTORY LOW-LEVEL APIs

VMP interfaces to the sound hardware by means of a set of hardware-specific, low-level APIs called VAPI, SAPI and AAPI.  VMP provides a small set of powerful function calls that are used for high-order tasks, such as simultaneously playing MIDI and digital audio files.  The low-level APIs, in turn, address the MIDI interface, synthesis and digital audio hardware directly.  In other words, VMP makes calls to the low-level APIs, which directly control the sound hardware.

Some multi-function sound cards use a combined low-level API called TAPI that includes the MIDI, FM and Digital Audio functions.

The vast majority of sound-related applications can be accomplished with the functionality provided by VMP.  However, in certain situations an application may have to access or control the sound hardware in a manner not provided by the VMP.  For example, your application may need to receive incoming MIDI data, utilize a Digital Audio file format not supported by VMP, etc.

In such cases where more direct hardware control is required, Voyetra's development toolkit contains the necessary documentation and support files for making calls to the low-level APIs.  If you're writing an application that needs to do this, contact Voyetra Technologies for details.

### 2.5.1	VAPIXE "VAPI"§ Drivers For MIDI Hardware

The file size for each VAPI version varies between 8K - 11K.

The Voyetra Application Program Interface (VAPI)  provides a standardized set of MIDI-specific function calls based on the CAPI specification established for the Yamaha C1 Music Computer.  All of the features supported by a particular MIDI interface hardware are incorporated into its VAPI driver.

### 2.5.2	SAPIXE "SAPI"§ Drivers For PC Sound Cards

The file size for each SAPI version varies between 5K - 16K.

The Synthesis Application Program Interface (SAPI) is an extension to VAPI that allows an application to control voices on a sound card as if they were MIDI synthesizers.  SAPI provides 128 preset sound programs which can be accessed via MIDI program numbers.  SAPI's patch, channel and drum note mapping conform to the General MIDI standard and Microsoft's Authoring Standards for MIDI Files.

SAPI turns your PC sound card into a "pseudo-MIDI" instrument by providing it with a MIDI implementation.  This allows VAPI to access all of the voice parameters and transmit MIDI note information as if it were controlling an external MIDI synthesizer on a separate MIDI port.

In situations where a PC sound card and MIDI interface hardware are both installed in the PC, VAPI differentiates between the external and internal MIDI routing by assigning different "virtual MIDI ports" to each one.  For instance, external MIDI devices may be

controlled as port 1 while the PC sound card's synth is controlled via port 2.  SAPI can also address multiple sound cards installed within the same PC.

### 2.5.3	AAPIXE "AAPI"§ Drivers For Digital Audio Hardware

The file size for each AAPI version varies between 14K - 20K.

The Audio Application Program Interface (AAPI) provides a set of function calls for controlling digital audio record/playback hardware.

This low-level API moves data to and from RAM buffers while providing callback functions so that higher-level functions can keep the buffers filled.  Since AAPI works to and from RAM, it makes no assumptions about file formats, disk access methods or multitasking solutions.  Each AAPI version supports the native bit resolution and sample rate of the hardware it is designed for.

### 2.5.4	Combined API Versions (TAPI)XE "TAPI"§

The file size for each TAPI version varies between 10K - 30K.

Combined APIs are also available for PC sound cards that offer combinations of MIDI, synthesis and digital audio functions.  By combining the VAPI, SAPI and AAPI for a multi-function card, a Total Application Program Interface (TAPI) offers reduced size, thus conserving RAM space.

For instance, the VAPI, SAPI and AAPI drivers for a Sound Blaster use about 40K when loaded separately, as compared to the combined TAPISB, which uses under 25K.  This is nearly 40% smaller than the individual APIs.

In general, if you're using the same card for MIDI and digital audio functions, you should use the TAPI driver for that card.

For a complete version of the Sound Factory Developer's Kit and licensing information, contact Voyetra Technologies at:

(914) 738-4500   9 AM - 5 PM  E.S.T.

XE "Voyetra Sound Factory" \r "VoyetraSF"§

< This page intentionally left blank >