Dec 18,  1996

YAMAHA Corporation

The description YAMAHA OPL3-SA Series Driver for WindowsNT

Version3.02 Driver

1. Supported OS and Hardware

- 1）OS

WindowsNT 3.51/3.5   (English)

WindowsNT 4.0   (English)

- 2）Hardware

YMF701/YMF701B(OPL3SA) /YMF711/718(OPL3SA2) /YMF715/719(OPL3SA3)

2. Supprted devices

- １）Wave-In/Out Device

１．Supported sampling rate ：48.0/44.1/37.8/33.075/32.0/27.42/22.05

18.9/16.0/11.025/9.6/8.0/6.62/5.51  kHz

２．Format

ADPCM(4-bit IMA compatible)

μ-Law(8-Bit Companded)

A-Law(8-Bit Companded)

Linear(8-Bit unsigned/16-Bit two’s complement, Big Endian)

Other compressed format is supported by compressed Audio CODEC.

３．FullDuplex

FullDuplex is supported by setting 0n to“Use Dual DMA” in the case of

same sampling rate.

- ２）MIDI-In Device

MIDI-In Device：MPU401 midi input(UART Mode)

- ３）MIDI- Out Device

Two MIDI devices listed below are supported by this driver.

１．FM synthesizer(OPL3)

２．MPU401 midi output(UART Mode)

- ４）Mixer Device

１．PlaybackVolume

Stereo：Master/Wave/ Synthesizer(FM/external synthesizer) /CD audio/ LineIn

Mono ： MicIn/MonoIn(MonoIn is not supported on OPL3-SA)

２．RecordingVolume

Stereo： CD audio/LineIn/Loopback

Mono ： MicIn

- ５）Joystick

Joystick is not supported for WindowsNT3.51.

For WindowsNT4.0,Joystick is supported by installing Analog JoystickDriver.

（Port:201h)

- ６）DirectSound

DirectSound is not supported for WindowsNT3.51.

For WindowsNT4.0,DirectSound is supported unless WaveDevice is used at the same

time.

3. Others

- １）APM

APM cannot be supported by function of WindowsNT3.51/4.0.

- ２）Plug&Play

Plug&Play is not supprted for WindowsNT 3.51.

For WindowsNT 4.0,Plug&Play is supported by using PnP-BIOS’s information of

registry.

- ３）Resource

Listed below is the possible resources that the driver might use.

１．WSS(Wave-In/Out,Mixer)

I/O Port： 530h,604h,E80h,F40h

IRQ   ： 5(not supported on OPL3-SA),7, 9, 10, 11

DMA  ： 0, 1, 3

Full Duplex : <Out 0 In 1>,<Out 1 In 0>,<Out 3 In 1> (OPL3-SA)

In’s DMAchannel must be difference from Out’s DMA

channel(OPL3-SA2,3)

２．MPU401

I/O Port： 300h, 332h, 334h, 330h

IRQ   ： 5, 7, 9, 10, 11(OPL3-SA2,3)

３．OPL3

I/O Port： 388h

Dec 18,  1996

Installation of OPL3-SA Series Driver for WindowsNT

Version3.02

The setup disk includes the following files.

- oemsetup.inf

- opl3sa.dll

- opl3sa.sys

- midimap.cfg

1.   How to install the driver

WindowsNT3.51

(1) Start  Microsoft WindowsNT Version3.51.

(2) Logon as Administrator Class user.

(3) Double click  “Drivers” in the control panel.

(4) Click “Add” .

(5) Double click  “Unlisted or Updated Disk” in list.

(6) Insert the floppy disk provided by Yamaha.

(7) Type the drive where the floppy disk is inserted.

(8) Click “OK”

(9) Set Parameters, 	look next 2.DriverSetup section.

(10) Shutdown and Restart your Computer.

WindowsNT4.0

(1) Start  Microsoft WindowsNT Version4.0 or later.

(2) Logon as Administrator Class user.

(3) Double click  “Drivers” in the control panel.

(4) Click “Add” .

- Double click  “Unlisted or Updated Disk” in list.

- Insert the floppy disk provided by Yamaha.

(6) Type the drive where the floppy disk is inserted.

(7) Click “OK”

(8) Set Parameters, 	look next 2.DriverSetup section.

(9) Shutdown and Restart your Computer.

2.   Driver Setup

When you install the driver or  run driver’s setting, the following Driver Configuration Box is shown.

Release note for the OPL3-SA Series WinNT driver

# 1.Version information and date

supported OS:WinNT ver3.5/3.51/4.0

supported hardware: YMF701/YMF701B(OPL3-SA)/YMF711(OPL3-SA2) /YMF715(OPL3SA3)

Version:302

File name:opl3sa.sys (12/12/96,71808byte)

opl3sa32.dll (12/12/96,89600byte)

oemsetup.inf (7/17/96,542byte)

midimap.cfg (10/5/96,56418byte)

# 2.Difference from version 3.00

¥Changed the name of line,”AUX” into “CD” on recording.

¥DDK’s library for version3.51 was changed to version4.00,when making the driver.

By this modification, the form of driver’s installing Dialog was changed to the

form of WindowsNT4.0.

¥The logic to reset IRQ of MPU401 and SB was added in order to modify hardware’s bug.

¥Mono-in was not shown for OPL3-SA board.

# 3.Bug list

・This driver failed on ACT-Test’s WaveOut Case64 and Mixer Case 10.

About this FAIL, we are making an inquiry to Microsoft because NT   standard Audio Driver fails at same test.