*** Release Notes for HIT-IDE.SYS Ver.1.06d2 ***

1. General

HIT-IDE.SYS is the device driver for DOS CD-ROM Extensions that supports

ATAPI CD-ROM drives.

2. Command line switches of 'HIT-IDE.SYS'

(1) SYNTAX

DEVICE = [x:][\dir\]HIT-IDE.SYS /D:(device name) [/N:(number of drives)]

[/T:(drive type)] [/S:(base I/O address)] [/V]

(2) Meaning of switches

/D:(device name)         Specifies "Device Name".

/N:(number of drives)    Specifies number of drives to be supported

by this driver.  Values 1 to 8 are valid.

If this switch is omitted, the driver will

search for all connected drives.

NOTE:  To speed the booting process, use

/N:1 if you only have one drive connected.

The search will stop after the first drive

is found.

/T:(drive type)          Specifies the first drive type to begin to

support.

Definition of drive types are as follows:

/T:   base I/O address    master/slave

---------------------------------------------

0     1F0h(primary port)  master

1     1F0h(primary port)  slave

2     170h(2nd port)      master

3     170h(2nd port)      slave

4     1E8h(3rd port)      master

5     1E8h(3rd port)      slave

6     168h(4th port)      master

7     168h(4th port)      slave

If this switch is omitted, it is the same as

/T:0.

NOTE:  To speed the booting process, use

/T:1 if you know your drive is connected

to the primary slave.  Use /T:2 if your

drive is connected to the secondary master.

The search will start at this number and

continue until /N:(number of drives) are

found.

/S:(base I/O address)    Change IRQ number to the specified base I/O

address.

base I/O           IRQ No.

address    without /S:    with /S:

(default)    (alternate)

----------------------------------------------

170h            15              10

1E8h            12              11

168h            10              9

(1F0h)          (14)            (14)

/V                      Display installation information.

NOTE:  Use the /V option to show you where

your drive(s) are connected.  Then you can

use the /T: and /N: options to speed your

booting process in the future.

3. Examples of installation

<Case 1> -- Default is automatic search for all drives connected.

DEVICE = HIT-IDE.SYS /D:MSCD001

-- Install all drives connected to primary, 2nd, 3rd & 4th.

<Case 2> -- Using /N: switch to limit the maximum number of drives.

DEVICE = HIT-IDE.SYS /D:MSCD001 /N:2

-- Install maximum of 2 drives while auto searching,

beginning the search with Primary Master.

<Case 3> -- Using /T: switch to change the start of automatic search.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2

-- Search beginning at /T:2 (2nd port and master).  Continue

searching through /T:7.

<Case 4> -- Using /N: and /T: switches together.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:2

-- Search beginning at /T:2 installing maximum 2 drives.

<Case 5> -- A single drive is connected to Primary Slave.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:1 /N:1

-- Search beginning at /T:1 installing only 1 drive.

<Case 6> -- A single drive is connected to Secondary Master.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:1

-- Search beginning at /T:2 installing only 1 drive.

<Case 7> -- Using /S: switch to use alternate IRQ numbers.

DEVICE = HIT-IDE.SYS /D:MSCD001 /S:170,168

-- Auto search all drives using alternate IRQ numbers

for 170 (IRQ 10) and for 168 (IRQ 9).   Use default

IRQ numbers for others (1F0 IRQ 14; 1E8 IRQ 12).

<Case 8> -- Using all switches together.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2 /N:1 /S:170

-- Install only 1 drive starting search at 2nd port master.

Use alternate IRQ of 10 for 170.

4. For Windows 95

If you are using one of our IDE CD-ROM drives on a DOS/Windows 3.x system

and you upgrade your system with Windows 95, Windows 95 will automatically

upgrade your system appropriately and the drive will work well.

If you are using Windows 95 and then adding one our IDE CD-ROM drives,

please run the "CDSETUP.EXE" program from the installation disk for the

drive under Windows 95 and restart your system.  After this, the drive

will work well.

The above specifications are subject to change.

May 10, 1996

Hitachi America, Ltd.