Driver Installation Guide for Microsoft Windows NT 3.5

============================================================

Introduction

============

This document describes the procedures to install  Windows NT 3.5

driver for Plug&Play Ethernet Adapter.

MS Windows NT 3.5 OS doesn't support Plug&Play feature and if your

computer does not support PnP function also,  you must set the LAN

card in "Jumperless" mode through our PNPSETUP program; Otherwise,

it can not work correctly.

Installation Procedures

=======================

1. If Windows NT operating system is being installed, select other

manufacturers from the network adapter card options and follow

steps 3-9.

2. If Windows NT operating system is already installed, logon as

Administrator and select Netwrok icon from the control panel.

3. In the Network Setting dialog box, choose "Add Adapter"button.

The Add Network Adapter dialog box appears.

4. In the Add Network Adapter dialog box, pull the adapter card list

menu and select "<Other>Requires disk from manufacturer" from the

list, and then press <Enter> button. .

5. Insert the driver diskette in Drive A/B.

6. In the netx windows, specify the correct drive and path (A:\WINNT35

or B:\WINNT35) when asked to give the source for the driver, and

then choose the OK button.

7. Then, select the "Plug&Play Ethernet Adapter" from the dialog box

to install adapter.

8. The screen will appear "Input EthernetID" dialog box which is provide

by PNPNT.SYS driver. This option is only required when you have more

than one Plug&Play Ethernet Adapters on this computer. Select SKIP

if only one adapter is installed on this computer.

9. Finally close the Setup dialog box and exit from the Add Network

adapter dialog box to complete the installation. Follow any

instructions appearing on the screen.

10. Shutdown and reboot the system.