# Using the Configuration Program to configure your LCS-8634L

This section describes how to use the Configuration/Diagnostic Program

named 'LSCFG.EXE' which is contained within the root directory of the utility

diskette we provide you to configure the hardware settings of your

LCS-8634L.

The factory setting of your LCS-8634L are as following:

Board Type	: NE2000 (16 bit)

Medium Type	: 10Base-T/10Base-2 (AUTO)

(All of LCS-8634L series except LCS-8634L REV.D1)

10 Base-2

(LCS-8634L REV.D1)

I/O Base Address : 300H

IRQ Number	: IRQ 3

Remote Boot ROM	: C8000H (8K)

Zero Wait State	: Disable

We recommend that after you install the LCS-8634L to your PC, please

excute LSCFG.EXE, the configuration program. LSCFG.EXE can detect that if

your LCS-8634L conflict with other I/O device or not. If conflict exists,

LSCFG.EXE will reconfigure your LSC-8634L to another available I/O base

address, IRQ number, Boot ROM address ... etc. .

IMPORTANT:

DO NOT excute LSCFG.EXE after loading the LAN CARD DRIVER of your

Network Operating System because LSCFG.EXE will re-initialize LCS-8634L.

This will causes the LAN CARD DRIVER fails to work and unpredictable

troubles.

# Excute this program

To excute the program, just specify the filename following the dos prompt.

For example:

A>lscfg <enter>

Then some message will be shown on your screen as follows:

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������ͻ

��Board�Setup���������

� Run Diagnostic	  �

� View System Status �Status �������������������������������Ŀ

� Exit		  �al ID      : 080000001075		  �

��������������������ͼType       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H			  �

� IRQ Number       : IRQ 3			  �

� Remote Boot ROM  : C8000H (8K)		  �

� Zero Wait State  : Disable 		  �

� Model Name       : LCS-8634L REV.D1	  �

������������������������������������������������

Esc-Exit	      ##-Select 	      #��-Make choice

Two windows are opened on the background, and active window is drown

with highlight color (yellow color on VGA) double line. Now you can use

the function key shown at the bottom of your screen to move cursor and

select the item you desire.

There are four options in the main menu:

o Board setup --------- To change the hardware configuration.

o Run Diagnostic ------ To perform a series of hardware tests.

o View System Status -- To view the used and unused I/O, IRQ and

memory of your PC.

o Exit ---------------- To terminate this program.

# Board Setup

From the Main Menu, select the "Board Setup".

The Main Menu window goes behind the Board Status window, and the

Board Status window becomes active window.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�Setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������ͻ

� Exit	   ��Physical�ID������:�080000001075���������������

�������������ĺ Board Type       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H			  �

� IRQ Number       : IRQ 3			  �

� Remote Boot ROM  : C8000H (8K)		  �

� Zero Wait State  : Disable 		  �

� Model Name       : LCS-8634L REV.D1	  �

����������������������������������������������ͼ

Esc-Exit	       ##-Select	      #��-Make choice

The available function keys are shown at the bottom of the screen.

The description of items of Board Status Menu are as follows:

- Physical ID:

The 12 digit number is the Physical ID of your LAN Card.

You can modify this number by selecting this item. (We recommend

you NOT to change this value if not necessary.)

- Board Type:

The type of LAN card that your LCS-8634L is campatiable with.

You can select this item to change the Board Type.

- Medium Type:

The network interface of your LCS-8634L currently using. You can

change it by selecting this item.

- I/O Base Address:

The first I/O Address where your LCS-8634L currently occupies.

The LCS-8634L will occupiy 16 continous I/O port. You can select

this item to change the I/O base.

- IRQ Number:

The IRQ line of the ISA slot which your LCS-8634L currently using.

To change IRQ Number, just select this item.

- Remote Boot ROM:

It shows the Base Address and Size of memory where the LCS-8634L

locates your Boot ROM. If you have a Remote ROM installed on the

"Boot ROM Socket" of the LCS-8634L, the contents of this Boot ROM

will be mapped to this memory area.

You can change the Base Address and Size by selecting this item.

- Zero Wait State:

This option allows you to force each Read/Write command involved

with your LCS-8634L to operate under zero-wait state.

We recommand you to perform a diagnostic after you enable this

option to ensure the LCS-8634L can work properly under Zero-Wait

State on your PC because of the deviation of Signal Timing of

different PC chip set.

- Model Name:

It shows the Model Name of your LAN card. The LSCFG.EXE version

2.10 can distinguish the Model of your LCS-8634L series LAN card by

reading EEPROM data.

This field can not be modified.

o Physical ID:

To modify the physical ID of LCS-8634L, select "Physical ID" from the

Board Status menu, the Phycal ID window will appear on your screen.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	  (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�Setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������ͻ

� Exit	   ��Physical�ID������:�080000001075���������������

�������������ĳ Board Type       : NE2000 (16 bit) 	  �

� Medium� Physical ID ��������������������������������ͻ

� I/O Ba�	The Physical ID is the address of the node�

� IRQ Nu� on the network. Each incoming packet will be �

� Remote� accepted or be rejected by the node according�

� Zero W� to this number.				  �

� Model �	We recommend that DO NOT change this value�

�������ĺ if not necessary. A wrong setting will cause �

� troubles on your network.			  �

�						  �

�   Physical ID : 080�00001075		  �

����������������������������������������������ͼ

Esc-Previous  # -Move cursor  0..9,A..F-Input HEX number  #��-Confirm change

Now, you can input a 12-digit Hex number to replace the orginal ID.

IMPORTANT:

A standard IEEE 802.3 packet kas a Destination address field that

indicates the destination of the packed on the network and is used to

filter unwanted packets from reaching a node.

The LCS-8634L compares the destination address field of the incoming

packet to its Physical ID to determine whether to accept this packet or

not.

We recommand you NOT to change this value if not necessary. A wrong

setting will cause serious troubles on your network.

o Board Type

The LCS-8634L is conpatible with three kinds of LAN card that are well-

known all over the world:

1. Novell NE2000

2. Novell NE1000

3. Western digital WD8013

To change the board type of your LCS-8634L, select "Board Type" from

Board Status menu.

The Board Type window appears.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�Setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������Ŀ

� Exit	   � Physical ID      : 080000001075		  �

�������������ĳ�Board�Type�������:�NE2000�(16�bit)������������

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H	   � Board Type ����ͻ

� IRQ Number       : IRQ 3	   ��NE2000�(16�bit)��

� Remote Boot ROM  : C8000H (8K)� NE1000 ( 8 bit) �

� Zero Wait State  : Disable    � WD8013 (16 bit) �

� Model Name       : LCS-8634L R�����������������ͼ

������������������������������������������������

Esc-Exit	       ##-Select		#��-Make choice

Use the function keys shown at the bottom of the screen to select the

board type you desire.

o Medium Type

You have four choices of medium type of LCS-8634L:

1. 10Base-T/10Base-2 Auto-detect

If you select this option, your LCS-8634L can select the medium

interface -- BNC port or RJ45 phone jack which is connected to the

cable of your network, automatically.

2. 10 Base-2

Set the BNC connector to be the medium interface of the

LCS-8634L.

3. 10 Base-5

Set the AUI port to be the medium interface of the LCS-8634L.

To change the medium type of LCS-8634L, select "Medium Type" from Board

Status menu.

The Medium Type window appears.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�Setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������Ŀ

� Exit	   � Physical ID      : 080000001075		  �

�������������ĳ Board Type       : NE2000 (16 bit) 	  �

��Medium�Type������:�10Base-2�(Thin�ethernet)���

� I/O Base Address : 300H			  �

� IRQ Number       : IRQ 3			  �

� Re� Medium Type �������������ͻ		  �

� Ze� 10Base-T/10Base-2 (AUTO)  �		  �

� Mo��10Base-2�(Thin�ethernet)���V.D1	  �

���ĺ 10Base-5 (Thick ethernet) ����������������

���������������������������ͼ

Esc-Exit	       ##-Select		#��-Make choice

Use the function keys shown at the bottom of the screen to select the

appropriate medium type.

To avoid that you may select the media type which the board doesn't

support, the unavailable option(s) is(are) shown with gray color, and the

cursor skips them.

o I/O Base Address

To change the I/O base address setting of your LCS-8634L, from the Board

Status menu, select "I/O Base Address".

The I/O Address window appears.

����������������������������������������������������������������������������ͻ

�	    ShineNET LCS-8634L Setup & Diagnostic Program  V1.00	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ			      � I/O Address ��ͻ

��Board�setup���������			      �   200H -- 21FH �

� Run Diagnostic	   �			      �   220H -- 23FH �

� View System � Board Status ������������������ĺ   240H -- 25FH �

� Exit	    � Physical ID      : 080000001021 � # 260H -- 27FH �

�������������ĳ Board Type       : NE2000 (16 bi�   280H -- 29FH �

� Medium Type      : 10Base-2 (Thi�   2A0H -- 2BFH �

��I/O�Base�Address�:�300H����������   2C0H -- 2DFH �

� IRQ Number       : IRQ 3	      � # 2E0H -- 2FFH �

� Remote Boot ROM  : C8000H (8K)  ����300H�--�31FH��

� Zero Wait State  : Disable      �   320H -- 33FH �

� Model Name       : LCS-8634L REV�   340H -- 35FH �

���������������������������������ĺ   360H -- 37FH �

�   380H -- 39FH �

� # 3A0H -- 3BFH �

� # 3C0H -- 3DFH �

� # 3E0H -- 3FFH �

����������������ͼ

Esc-Exit	      ##-Select 	       #��-Make choice

Use the function keys shown at the bottom of the screen to select the

appropriate I/O base address.

To avoid the setting that may conflict with other board, the I/O address

have been occupied by other devices are marked with a smile face sign and

shown with gray color, and the cursor skips them.

o IRQ Number

To change the IRQ number setting of your LCS-8634L, from the Board Status

menu, select "IRQ Number".

The IRQ Number window appears.

���������������������������������������������������������������������������ͻ

�	   ShineNET LCS-8634L Setup & Diagnostic Program  V1.00 	    �

�	(C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	    �

���������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�setup���������

� Run Diagnostic	  �

� View System � Board Status ������������������������Ŀ

� Exit	   � Physical ID      : 080000001021	   �

�������������ĳ Board Type       : NE2000 (16 bit� IRQ Number ����ͻ

� Medium Type      : 10Base-2 (Thin����IRQ�Level��3���

� I/O Base Address : 300H	      � # IRQ Level  4	�

��IRQ�Number�������:�IRQ�3����������   IRQ Level  5	�

� Remote Boot ROM  : C8000H (8K)   �   IRQ Level  9	�

� Zero Wait State  : Disable       �   IRQ Level  10 �

� Model Name       : LCS-8634L REV.�   IRQ Level  11 �

����������������������������������ĺ   IRQ Level  12 �

�   IRQ Level  15 �

�����������������ͼ

Esc-Exit	      ##-Select 	       #��-Make choice

Use the function keys shown at the bottom of the screen to select the

appropriate IRQ number.

To avoid the setting that may conflict with other board, the IRQ have

been occupied by other devices are marked with a smile face sign and shown

with gray color, and the cursor skips them.

o Remote Boot ROM

The LCS-8634L provides four different sizes of Remote Boot ROM:

8K, 16K, 32K, 64K

The range of ROM base address is from C0000H to DE000H. The number of

ROM base option depend on the size of the Boot ROM. If you choose 8K as

your Boot ROM size, the number of the base address option will be up to 16.

If you choose 64K as your Boot ROM size, the number of the base address

option is 2 only.

To change the Boot ROM setting of your LCS-8634L, from the Board Status

menu, select "Remote Boot ROM".

The ROM Size window will appear first to ask you to set the Boot ROM size.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������Ŀ

� Exit	   � Physical ID      : 080000001075		  �

�������������ĳ Board Type       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H			  �

� IRQ Number       : IRQ 3			  �

��Remote�Boot�ROM��:�C8000H�(8K)����������������

� Zero Wait State  : Disable 		  �

� Mode� ROM Size ����ͻS-8634L REV.D1	  �

�����ĺ��8�Kilo-Bytes���������������������������

� 16 Kilo-Bytes �

� 32 Kilo-Bytes �

� 64 Kilo-Bytes �

� NO Boot ROM	 �

���������������ͼ

Esc-Exit	      ##-Select 	       #��-Make choice

Use the function keys shown at the bottom of the screen to select the

size of the Boot ROM.

After this step, the ROM Address window is opened on your screen.

In this example, the Boot ROM Size is 8K.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ		   � ROM Address ������ͻ

��Board�setup���������		   � # C0000H -- C1FFFH �

� Run Diagnostic	  �		   � # C2000H -- C3FFFH �

� View System � Board Status ��������ĺ   C4000H -- C5FFFH �Ŀ

� Exit	   � Physical ID      : 080�   C6000H -- C7FFFH � �

�������������ĳ Board Type       : NE2����C8000H�--�C9FFFH�� �

� Medium Type      : 10B�   CA000H -- CBFFFH � �

� I/O Base Address : 300�   CC000H -- CDFFFH � �

� IRQ Number       : IRQ�   CE000H -- CFFFFH � �

��Remote�Boot�ROM��:�C80�   D0000H -- D1FFFH � �

� Zero Wait State  : Dis�   D2000H -- D3FFFH � �

� Mode� ROM Size ����ͻS�   D4000H -- D5FFFH � �

�����ĺ��8�Kilo-Bytes��ĺ   D6000H -- D7FFFH ���

� 16 Kilo-Bytes � �   D8000H -- D9FFFH �

� 32 Kilo-Bytes � �   DA000H -- DBFFFH �

� 64 Kilo-Bytes � �   DC000H -- DDFFFH �

� NO Boot ROM	 � �   DE000H -- DFFFFH �

���������������ͼ ��������������������ͼ

Esc-Exit	      ##-Select 	       #��-Make choice

Use the function keys shown at the bottom of the screen to select the

base address of the Boot ROM.

To avoid the setting that may conflict with other board, the memory blocks

have been occupied by other devices are marked with a smile face sign and

shown with gray color, and the cursor can not move on them.

o Zero wait

To enable or disable the wait state, just move the cursor on "Zero Wait

State" and press Enter Key. The status of this option will switch to another,

either Enable or Disable.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������ͻ

� Exit	   � Physical ID      : 080000001075		  �

�������������ĺ Board Type       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H			  �

� IRQ Number       : IRQ 3			  �

� Remote Boot ROM  : C8000H (8K)		  �

��Zero�Wait�State��:�Disable��������������������

� Model Name       : LCS-8634L REV.D1	  �

����������������������������������������������ͼ

Esc-Exit	      ##-Select 	       #��-Make choice

# Save your setting into the EEPROM of the LCS-8634L

When you finish the configuration of your LCS-8634L, press ESC key to

exit this menu.

If you have maked some changes, the LSCFG.EXE will give you some messages

before you back to Main Menu -- a Warning window appears on your screen, and

asks you if you want to save the new configuration into the EEPROM of your

LCS-8634L or not.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

��Board�setup���������

� Run Diagnostic	  �

� View System � Board Status �������������������������������Ŀ

� Exit	   � Physical ID      : 080000001075		  �

�������������ĳ Board Type       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-5 (Thick ethernet) �

� I/O Bas� Save New Configuration �������������������ͻ

� IRQ Num�	 The Configuration of your LCS-8634L had �

� Remote � been modified, do you want to save the new �

� Zero Wa� Configuration to the EEPROM ?		 �

� Model N�						 �

��������ĺ	 �Yes,�save�it��     Oh! No, no...	 �

�						 �

��������������������������������������������ͼ

Y-Yes	      N-No	   # -Select	     #��-Make choice

Use the function keys shown at the bottom of the screen to make your

choice.

# Run Diagnostic

From the Main Menu, select "Run Diagnostic".

The Diagnostic window appears. After seconds of performing self-testing,

the results of the diagnostic are shown in this window.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ

� Board setup	  �

��Run�Diagnostic������

� View System Status �Status �������������������������������Ŀ

� Exit		  �al ID      : 080000001075		  �

����������������������Type       : NE2000 (16 bit) 	  �

� Medium Type      : 10Base-2 (Thin ethernet)  �

� I/O Base Address : 300H			  �

� IRQ Num� Diagnostic ��������������������������������ͻ

� Remote � Internal Register test............. Pass	  �

� Zero Wa� EEPROM test........................ Pass	  �

� Model N� Buffer test (I/O mode)............. Pass	  �

��������ĺ Loop Back Mode 1 test ............. Pass	  �

� Loop Back Mode 2 test ............. Pass	  �

� Cable connection test ............. Testing �

���������������������������������������������ͼ

After this diagnostic, you can press ank key to back to Main Menu.

# View System Status

From the Main Menu, select "View System Status".

Several windows appear on your screen and will show out occupied I/O

addresses, IRQ numbers and Memory blocks respectively.

The places by other devices are marked with a smile face sign. The place

where occupied by your LCS-8634L is marked with a asterisk.

����������������������������������������������������������������������������ͻ

�	     ShineNET LCS-8634L Setup & Diagnostic Program V2.10	     �

�	 (C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	     �

����������������������������������������������������������������������������ͼ

� Main Menu ��������Ŀ    � I/O Occupied �ͻ  � UMB Occupied �����ͻ

� Boa� IRQ Occupied ��ͻ  �   200H -- 21FH �  � # C0000H -- C1FFFH �

� Run� * IRQ Level  3  �  �   220H -- 23FH �  � # C2000H -- C3FFFH �

� Vie� # IRQ Level  4  �at�   240H -- 25FH ��ĺ   C4000H -- C5FFFH �

� Exi�   IRQ Level  5  � I� # 260H -- 27FH �07�   C6000H -- C7FFFH �

����ĺ   IRQ Level  9  �pe�   280H -- 29FH �6 � * C8000H -- C9FFFH �

�   IRQ Level  10 �yp�   2A0H -- 2BFH �(T�   CA000H -- CBFFFH �

�   IRQ Level  11 � A�   2C0H -- 2DFH �  �   CC000H -- CDFFFH �

�   IRQ Level  12 �er� # 2E0H -- 2FFH �  �   CE000H -- CFFFFH �

�   IRQ Level  15 �oo� * 300H -- 31FH �K)�   D0000H -- D1FFFH �

�����������������ͼt �   320H -- 33FH �  �   D2000H -- D3FFFH �

� Model Name�   340H -- 35FH � R�   D4000H -- D5FFFH �

� Symbol ��������ͻ�ĺ   360H -- 37FH ��ĺ   D6000H -- D7FFFH �

�   Free	    �  �   380H -- 39FH �  �   D8000H -- D9FFFH �

� * LCS-8634L     �  � # 3A0H -- 3BFH �  �   DA000H -- DBFFFH �

� # Other devices �  � # 3C0H -- 3DFH �  �   DC000H -- DDFFFH �

�����������������ͼ  � # 3E0H -- 3FFH �  �   DE000H -- DFFFFH �

����������������ͼ  ��������������������ͼ

You can press ank key to back to Main Menu.

# Message of confilct with other devices

LSCFG.EXE will give you some warning message if your LCS-8634L conflict

with other device. These message are described as following:

o I/O conflict

If your LCS-8634L use the same I/O address as other devices, LSCFG.EXE

will give you this message.

A Conflict I/O window is opened on the screen. The message contained

within this window explain that there is another device using the same I/O

address as your LCS-8634L, shows the I/O address and ask you would like to

let LSCFG.EXE choose an avaliable I/O for you or not.

���������������������������������������������������������������������������ͻ

�	   ShineNET LCS-8634L Setup & Diagnostic Program  V1.00 	    �

�	(C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	    �

���������������������������������������������������������������������������ͼ

� Conflict I/O ��������������������������������������ͻ

�	 There is another device in your PC system using �

� the same I/O address which your LCS-8634L currently �

� using (280H - 29FH). This situation would make your �

� PC system and LCS-8634L fail to work.		 �

�	 In order to keep your system works correctly,	 �

� you can do the followings :			 �

�							 �

�	 Would you like ...				 �

� - Let LSCFG.EXE relocate your LCS-8634L to another  �

�   available I/O address.				 �

�							 �

�	 or ... 					 �

� - Exit LSCFG.EXE ,power-off your PC system, and	 �

�   remove the device which conflict with LCS-8634L.  �

�							 �

�	  Please use ## key & #�� key to make choice.	 �

�����������������������������������������������������ͼ

You can use the function keys shown in the window to make your choice.

The selected option is highlighted. Press Enter key will make the option

be chosen.

If you choose the first option -- Let LSCFG.EXE relocate your LCS-8634L

to another available I/O address, LSCFG.EXE will show another message to

tell you where the new I/O base addresses of your LSC-8634L is.

���������������������������������������������������������������������������ͻ

�	   ShineNET LCS-8634L Setup & Diagnostic Program  V1.00 	    �

�	(C)Copyright LONGSHINE Electronics Corp., 1992, 1993, 1994	    �

���������������������������������������������������������������������������ͼ

� Conflict I/O ��������������������������������������Ŀ

�	 There is another device in your PC system using �

� the same I/O address which your LCS-8634L currently �

� using (300H - 31FH). This situation would make your �

� PC system and LCS-8634L fail to work.		 �

� New I/O address ����������������������������������������������������ͻ

�  The I/O address of your LCS-8634L is relocated to 200H - 21FH.	  �

�									  �

�	 If you don't like this setting, you can select 'LCS-8634L Setup' �

�  from the Main Menu to change the I/O setting.			  �

�			  Press any key to continue .....		  �

����������������������������������������������������������������������ͼ

�	 or ... 					 �

� - Exit LSCFG.EXE ,power-off your PC system, and	 �

�   remove the device which conflict with LCS-8634L.  �

�							 �

�	  Please use ## key & #�� key to make choice.	 �

�������������������������������������������������������

o IRQ conflict

If your LCS-8634L use the same IRQ number as other devices, LSCFG.EXE

will give you this message.

A Conflict IRQ window is opened on the screen which just like Conflict

I/O window.  The message within this window explain that there is another

device using the same IRQ as your LCS-8634L, shows the IRQ number and ask

you would like to let LSCFG.EXE choose an avaliable IRQ for you or not.

If you choose the first option -- Let LSCFG.EXE reconfigure the IRQ

setting of your LCS-8634L to another available IRQ, LSCFG.EXE will tell

you the new IRQ of your LSC-8634L.

o Memory conflict

If your LCS-8634L use the same ROM address as other devices, LSCFG.EXE

will give you this message.

A Conflict Memory window is opened on the screen which just like Conflict

I/O window and Conflict IRQ window.  The message within this window explain

that there is another device using the same Memory Area as your LCS-8634L,

shows the address of the Memory Area and ask you would like to let LSCFG.EXE

choose an avaliable Memory Area for you or not.

If you choose the first option -- Let LSCFG.EXE choose another available

Memory Area for the Boot ROM of your LCS-8634L, LSCFG.EXE will tell you where

is the new Memory Area of your LSC-8634L.

---- End of file ----