This section describes how to install the Ethernet Pocket Adapter in

a workstation that uses DOS ODI. The ODI (Open Data-Link Interface)

allows you to:

* Expand your network by using multiple protocols without adding

extra network boards to the workstation.

* Communicate with a variety of workstations, file servers, and

main frame computers via different protocol stacks without rebooting

the workstation.

* Protect your investment by maintaining compatibility with future

technology. All protocols written to ODI specification can communicate

through any LAN adapter written to ODI specification.

* Have fewer hardware components to support, because a single LAN

driver can support several protocols.

NOTE: When using ODI shells, no files need to be generated.

CREATING A MASTER ODI WORKSTATION DISK

1. Format a disk using the FORMAT /s command.

2. Label the disk " Master ODI Workstation Disk "

3. Copy the appropriate SHELL file from the WS-GEN disk to the Master ODI

Workstation disk.

* NWTx.COM     loads into conventional memory.

* EMSNETx.COM  loads into expanded memory.

* XMSNETx.COM  loads into extended memory.

NOTE: x=3 if using DOS 3.3...x=5 if using DOS 5.0

4. Copy the following files from the DOSODI directory on this disk to the

Master ODI Workstation disk.

* LSL.COM

* NET.CFG driver for Ethernet Pocket Adapter

* PEAODI.COM LAN driver for Ethernet Pocket Adapter

* IPXODI.COM or other Protocol Stack file,

NETBIOS.EXE, INT2F.COM.

* RPLODI.COM use if disk is to be used for remote booting.

5. Create a CONFIG.SYS file on the master workstation disk using a DOS text

editor.

6. Add the executable files you have copied to the Master ODI Workstation disk

to the AUTOEXEC.BAT file. Load the files in the following order:

* Link support layer

* LAN drivers

* Protocol stacks

* The shell

The following is a sample AUTOEXEC.BAT file.

LSL

PEAODI

IPXODI

NET5

7. Login to the network with the Master ODI Workstation Disk to ensure that

files you created work properly with network.