SK-SNMPD v2.02

The SNMP directory has the following structure:

snmpread.doc

readme.mib

upps.asn

snmp.rel

-----TOOLS

-----AGENT

FTP

snmpread : this file

readme.mib : a short description of the implemented MIB

upps.asn : describes the MIB of SNMPD in ASN1 (for ASN1 compiler)

snmp.rel : release notes for SK-UPPS SNMP

In the subdirectories exists the following files :

AGENT	:	-SNMP-Agent     	:snmpd.exe

-configuration		:snmp.cnf

TOOLS	: 				:snmpwalk.exe

:snmpget.exe

-readme			:snmpwalk.doc

:snmpget.doc

:variables.doc

-configuration		:ip.cnf

:mib.txt

These are the SNMP-Daemon and SNMP-TOOLS for the UPPS drivers on Ethernet

Token Ring and FDDI. The private MIB is defined in the file 'UPPS.ASN'

which is in the directory SNMP on this disk.

The SNMPD implements the agent and has to run on each managed PC.

With the tool SNMPGET you can ask the agent for one specific variable (Get

Request), SNMPWALK implements the GET-NEXT-Request.

The SK-SNMPD is a TSR (Terminate and Stay Resident) program under

DOS which uses about 20kB memory. It can run on the three topologies

Ethernet, FDDI and Token Ring. On each a partially different MIB is

implemented. The SK-SNMPD checks at loading time on which topology

it is running an loads the according software.

All executables named above have their own UDP/IP

protocol stack implemented.

In the directory AGENT exists a subdirectory FTP which includes

a SNMPD using the UDP/IP protocol stack from FTP. Therefore this

SNMPD uses about 3kB memory less than the SNMPD with the own protocol stack,

but can only be used on top of FTP Software.

INSTALLATION OF SNMPD

---------------------

Since the SNMP agent should remain in the memory of the PC the whole time,

you should load the SNMP agent after the Data Link Interface Driver

but before any other UPPS client driver. The reason is, if

you want to unload the other UPPS client drivers sometimes, you should use

the reverse order. If you install the SNMP agent after all the other UPPS

client drivers, you would have to unload it every time when you want to unload

another UPPS client driver.

Use the SNMPD command line option '-c' (Chain mode), if you intend to install

another TCP/IP stack at the same time. The chain mode forces the SNMPD driver

to watch only for his own TCP/IP packets (with SNMP information) but to pass

through all other TCP/IP packets to the other TCP/IP stack.

Use the following commands in AUTOEXEC.BAT to load the UPPS SNMP agent:

UPPSDLI

SNMPD

[other UPPS clients]

Unload the SNMP agent with

SNMPD -u

if no other UPPS client is loaded after SNMPD .

PARAMETER / KONFIGURATION

-------------------------

The SK-SNMPD needs a configuration file which is per default in the current

working directory and named snmp.cnf. With the option -f you can specify

a different path and filename.

The configuration file looks like this:

host    	192.9.202.12

netmask 	255.255.255.0 (optional)

gateway 	192.9.202.4

sysContact      Supervisor Tel. 0721/792 0

sysName 	sk.ger.ka.test

sysLocation 	Schneider & Koch & Co, Siemensstr. 23, 76275 Ettlingen

host: specify the IP Adresse of this PC.

netmask: set the subnet bit mask.

gateway: specify the default Gateway; if an IP packet should be sent

to an other IP network you must specify this parameter.

sysContact: The textual identification of the contact person for

this managed node, together with information on how to

contact this person. (max. 80 characters)

sysName:    An administratively-assigned name for this managed node.

By convention, this is the node's fully-qualified domain

name. (max 40 characters)

sysLocation: The physical location of this node. (max 80 characters)

A line starting with "#" is handled as comment.

SUMMARY OF OPTIONS

------------------

-c: start SK-SNMPD in chain mode

-u: unload SK-SNMPD

-f: specify a configuration file (name and path)

-p: specify only another path for configuration file

-?: display usage