Unsupported Software !!

July 1992

LSB mode to MSB mode address conversion program

Use this program to calculate the address parameter (-a), when you

want to use the IBM LAN Support program on Ethernet.

You have to load a device driver called DXME0MOD.SYS in the file

CONFIG.SYS. As a parameter of this driver you have to input a

logical network address of the DOS Workstation. The address

is written in Token-Ring format:

device = dxme0mod.sys 400000001234

To use this address with the UPPSDLI driver, you have to convert it

into an Ethernet address format. This converted address has to be

written as a parameter of the UPPSDLI driver.

uppsdli -i5 -bcc -a02000000482c

Use the TOK2ETH.EXE program for an easy calculation of the

-a parameter of UPPSDLI in combination with IBM LAN Support program.

This software is unsupported !