WEFAX.EXE - Weather Facsimile Receiver (Version 1.5)

This weather facsimile program was written in order to provide a free, WEFAX

computer program capable of receiving WEFAX graphics without the price of

the available commercial wefax programs.

** This program is freeware!!! ** and may not be distributed for any price,

but should be distributed for free.

This program does require a special device that is capable of receiving

weather facsimile images.

This program cannot be decompiled or anything like that either.  It was

written by Jason Johnson and Todd Hansen, KD6YPS, for use by those wishing

to monitor wefax transmissions on HF frequencies.

If any portion of this documentation or the accompanying programs are

changed in any way, then it is illegal to distribute the software.

A user of this software should have knowledge in computers, TNCs, radio

receivers and WEFAX transmissions.

File Listing

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

WEFAX.DOC   - This file

WEFAX.EXE   - The program

VIEWFAX.EXE - A quick viewer for saved wefax images (This is not

required and has many errors).

[Usage: VIEWFAX <FileName> <CR>]

SAMPLE.ZIP  - A zip file containing some sample faxes recieved

Upgrades Over Previous Version

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1. You can select com ports 1 - 4 and all baud rates up to 28.8Kbps.

2. It has a better help file (Thank you Mario Campos).

3. It includes sample faxes

4. It has been tested more thoroughly.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

*** Special Notice ***

To use the print fax option, the TSR (terminate and stay resident)

program called GRAPHICS.COM (included with most versions of MS-DOS) must

be loaded.

COMMAND LINE PARAMETERS:

~~~~~~~~~~~~~~~~~~~~~~~~

The program accepts command line parameters as follows:

Usage:

From the command line: WEFAX [PORT#] [BAUDRATE]

Where: [BAUDRATE] = Speed to access the comport at (must be less than

or equal to 28800 baud and greater than 1200 baud).

[PORT#]    = # of comport to use.  (i.e. 1 for COM1, 2 for COM2,

etc.)

[/? or /HELP] = Show command line help/usage.

Note: If a baudrate or comport number are not given in the command line,

the program defaults to COM1 at 9600 Baud.

Also: Set data sample rate of your TNC to 1280 bit samples/sec.

On the Kantronics KPC-3, type the following at the "cmd:" prompt: WEFAX

1280 [CR]

------------------------------------------------------------------------------

This program was written for non-commercial distribution as freeware.  Since

we made this free, any problems encountered should be reported, but please

note that we are busy and may not be available to help you.

Send any comments, questions, descriptions of problems, or suggestions to:

Todd Hansen, KD6YPS

tshansen@sdcc10.ucsd.edu

Jason Johnson

johnson@fullcoll.bbs.edu

(No he isn't in college yet either)

Program Usage

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The program starts out in terminal mode with your TNC (Terminal node

controller). After you set-up your TNC to receive wefax data, press F1.

This will begin to display the graphic.

After you have received the image you want on the monitor screen, press any

key. This will bring up a display of a self explanatory menu. This allows you

to save the image, load another image and view it, print the current image

(using GRAPHICS.COM) and to exit back to terminal.

When in the terminal mode, you can press ESC and this will terminate the

WEFAX program. The WEFAX is not a very good terminal program and should only

be used to configure your TNC parameters to receive a WEFAX image.

Suggestions for receiving WEFAX images:

Read your owner's manual for your particular TNC and HF receiver.  The

receiver must be able to receive USB (Upper Side Band) transmissions. The

sample images were recieved using a KPC-3, a high end Radio Shack SW Reciever,

and an external long wire antenna. If your computer generates to much

interference you may want to put up an external antenna. It is a great

improvement.

Wefax Freq.

~~~~~~~~~~~~~~~~~~~~~~~~~

These are all the Wefax Freq. I have. They came from Bob Puharic, WF3H.

His recieveing location is Allentown PA. Not all stations are on 24 hrs/day.

All stations are USB. NAM is on 24 hrs/day. NAM sends satallite fotos at the

top of the hour.

Station               Freqs. (kHz)      Note

---------------------------------------------------------------

Un id'd        |      6918           |  Spain

NAM            |  3355, 8080, 10865  |  USN/USCG Maine

CFH            |  6496, 10536, 13510 |  Canada, Halifax

NPM            |  6453, 8680, 9088   |  USN, Pearl Harbor

Northwood      |      16912          |  UK

Bracknell      |  9203, 14436        |  UK

IMB55          |      8145           |  Rome

NMC            |  11122, 6906, 4622  |  USCG, Point Reyes, CA

WLO            |      9158           |  Mobile, AL

Nairobi, Kenya |      9044           |  Evenings

GFA23          |      8040           |  UK

Un id'd        |      7709           |  Weak, 2000 UTC

VFF/VFR        |      7710           |  Canada

RMC            |      11028          |  2000 UTC, Australia

GFA24          |   11086, 23333      |  Bracknell, UK

RKU71          |      13470          |  Moscow

US/NOAA        |6339, 4855.5, 3231.5 |

NOAA*          |      5906.4         |  0430 UTC

USAF*          |      18263.8        |  1400 UTC

USAF*          |      7868           |  0130 UTC

USAF*          |      2372.4         |  0530 UTC

* New freqs. from WF3H (Bob Puharic)

Also: Remember to use a 1280 bits per second sample rate.

Errors found so far

~~~~~~~~~~~~~~~~~~~~~~~~~~~

None.

Please send error reports.

Use Todd Hansen's address (tshansen@sdcc10.ucsd.edu)

it will be correct for a longer period of time.

Author's Note (Todd Hansen, KD6YPS)

~~~~~~~~~~~~~~~~~~~~~~~~~~~

I originally wrote this software because I had recently gotten a TNC

with the feature (KPC-3) and a SW Radio with USB reception. I wanted to have a

program that worked well and had instructions in english. That is the reason

that my friend and I teamed together to write this program. I decided to make

it free as an act of rebelion against a greedy oriented society and the fact

that I didn't want people having to pay money for it. I have recently become

very busy so I doubt you will ever see the famed version 2.0, but you don't

have to worry about having a mouse or a color monitor with version 1.X. I

have done most of this with a monochrome monitor.

Biography (Todd Hansen, KD6YPS)

~~~~~~~~~~~~~~~~~~~~~~~~~~

We are both High School Seniors (17 years old, 7/95) and into

computers a lot. We both also enjoy electronics and hiking. We both are doing

internships for our High School. My internship is setting up and maintaining

the novell network at school (that is a big job, especially with all the

politics) and createing the homepage for the City of Fullerton (It will be up

in a couple of weeks. Do a Lycos search). Jason will probably do his

internship on optics and alternate forms of digital communications. I am an

Eagle Scout and I enjoy camping and Backpacking alot. My amatuer callsign is

a technician, but I am trying to find time to learn the code. Yes, this has

been a paragraph of bragging, but I figure if we wrote this program we should

be allowed to brag. Adios.