__________________________________________________________________________

STACKER NOTE                              Stac Electronics Technical Note

Subject: Stacker coprocessor installation checklist.

Tech008 - 2/20/92

Page 1 of 1

__________________________________________________________________________

This note provides a checklist for troubleshooting the Stacker version 1.x

(8 bit) coprocessor, and the Stacker 2.0 XT/8 (8 bit) and AT/16 (16 bit)

coprocessor cards.

Symptoms:

A Stacker coprocessor installation exhibiting the following symptoms will merit

further investigation:

1. During Install, the coprocessor may fail to respond at all the addresses.

2. The Stacker.com message: <Coprocessor card not functioning. Invoking

software driver> may appear at boot time.

Solution:

First check the basics.

1. Check to ensure the card is properly seated in the slot.

2. Check to ensure the /B parameter in the Stacker driver contains a valid

address.  Possible base addresses are listed in the 2.0 manual pgs. 43-47

and on page 17 in the version 1 manual.

3. Run Install to test all addresses.

4. Try adding each address to the driver in Config.sys and COLD BOOT (Reset

or a power down/up).  A cold boot is necessary to flush addresses.

5. Try a different slot.

6. Try rearranging the order of the cards in the expansion slots.

7. Try disabling SHADOW RAM (Scratch RAM) if present.  This is done in the

BIOS setup at boot time or on some memory manager device lines in

Config.sys.

8. If possible, try the card in a 2nd machine to see if the problem is with

the card itself.

Card specific issues:

*XT/8: or a Stacker version 1 coprocessor card,

1. Check for address conflicts with a memory management program. Fixing

this problem usually involves adding memory exclude statements to the

memory management driver.  Try starting with the range C400-DFFF to

enable the testing of all possible addresses during Install.

2. If the XT/8  card is in a 386 machine, it may be that the motherboard

maps memory over the addresses that Stacker uses to access the card.

Refer to your system user's guide for more information.

3. Version 1.11 or prior with 16 bit VGA card, there may be a conflict

between the VGA card and the coprocessor.  Test the card with the VGA

card set to 8 bit mode.  If this eliminates the conflict, call STAC

technical support.

*AT/16

1. Something (usually another add-in card) is conflicting with the address

of your AT/16 card. Check the hardware addresses for your other cards

or change the hardware address for the AT/16 card.

2. The buss speed may be greater than 8 Mhz.  This value can sometimes be

set in the BIOS.  It is usually set as the clock speed (CLK) divided by

some number.  Try setting the speed to 8 Mhz or less to test. If buss

speed is the issue, call STAC technical support.

*MC/16

1. You must install and configure the MC/16 card before installing Stacker.

_________________________________________________________________________________

1992 STAC ELECTRONICS