TRAJECTORY MAKER

A Trajectory Simulation Tool

����������������������������

USER'S  GUIDE

ORBIT HORIZONS

94 Promenade

Irvine, California 92715

U.S.A.

COPYRIGHT NOTICE

This  document  and  software  package  consisting  of the

Trajectory  Maker and  Trajectory Scape  computer programs

are  copyrighted   (C)  1991,   1992  by   H.B.  Reynolds,

President, ORBIT  HORIZONS.  All rights  are reserved.  No

part of  this publication may  be reproduced, transmitted,

transcribed, stored in any retrieval system, or translated

into any language by any means without the express written

permission  of   ORBIT  HORIZONS,   94  Promenade,  Irvine

California 92715, USA.

SECOND EDITION/FIRST PRINTING

July 1992

ii

LIMITED WARRANTY

THIS USER'S  GUIDE, THE  PROGRAMS, "TRAJECTORY  MAKER" AND

"TRAJECTORY SCAPE" ARE SOLD  "AS IS",  WITHOUT WARRANTY AS

TO  THEIR  PERFORMANCE,  OR  FITNESS  FOR  ANY  PARTICULAR

PURPOSE.   THE   ENTIRE  RISK   AS  TO   THE  RESULTS  AND

PERFORMANCE OF THE PROGRAMS IS ASSUMED BY THE USER.

HOWEVER, ORBIT HORIZONS  WARRANTS THE MAGNETIC DISKETTE(S)

ON WHICH THE  PROGRAM IS RECORDED TO  BE FREE FROM DEFECTS

IN MATERIALS AND FAULTY WORKMANSHIP UNDER NORMAL USE FOR A

PERIOD  OF  NINETY DAYS  FROM  THE DATE  OF  PURCHASE.  IF

DURING THIS NINETY  DAY PERIOD THE  DISKETTE SHOULD BECOME

DEFECTIVE,  IT MAY  BE RETURNED  TO  ORBIT HORIZONS  FOR A

REPLACEMENT  WITHOUT CHARGE,  PROVIDED  YOU SEND  PROOF OF

PURCHASE OF THE PROGRAM.

YOUR SOLE AND EXCLUSIVE REMEDY IN THE EVENT OF A DEFECT IS

EXPRESSLY  LIMITED  TO  REPLACEMENT  OF  THE  DISKETTE  AS

PROVIDED  ABOVE.  IF  FAILURE OF  A DISKETTE  HAS RESULTED

FROM  ACCIDENT  OR  ABUSE, ORBIT  HORIZONS  SHALL  HAVE NO

RESPONSIBILITY TO REPLACE THE  DISKETTE UNDER THE TERMS OF

THIS LIMITED WARRANTY.

IN NO EVENT SHALL ORBIT HORIZONS  BE LIABLE TO YOU FOR ANY

DAMAGES, INCLUDING  BUT NOT  LIMITED TO  ANY LOST PROFITS,

LOST SAVINGS,  OR OTHER CONSEQUENTIAL  DAMAGES ARISING OUT

OF  THE  USE  OR  INABILITY  TO  USE  THESE  PROGRAMS  AND

ACCOMPANYING  MATERIALS EVEN  IF  ORBIT HORIZONS  HAS BEEN

ADVISED OF  THE POSSIBILITY  OF SUCH  DAMAGES, OR  FOR ANY

CLAIM BY ANY OTHER PARTY.

SOME STATES  DO NOT ALLOW  THE LIMITATION  OR EXCLUSION OF

LIABILITY FOR INCIDENTAL OR  CONSEQUENTIAL DAMAGES, SO THE

ABOVE LIMITATION OR EXCLUSION MAY  NOT APPLY TO YOU.  THIS

WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS, AND YOU MAY ALSO

HAVE OTHER RIGHTS WHICH VARY FROM STATE TO STATE.

THIS WARRANTY SHALL BE CONSTRUED, INTERPRETED AND GOVERNED

BY LAWS OF THE STATE OF CALIFORNIA.

YOU  AGREE TO  THESE TERMS  BY YOUR  DECISION TO  USE THIS

SOFTWARE.

iii

TABLE OF CONTENTS

WELCOME ............................................. viii

1  INTRODUCTION ........................................ 1

2  INSTALLATION ........................................ 3

3  TRAJECTORY MAKER INPUT DESCRIPTION .................. 7

3.1  Unit Convention Menu ......................... 7

3.2  Perturbation Model Option Menu ............... 7

3.2.1  Unperturbed Two-Body Option ......... 8

3.2.2  Gravity Harmonic Effects Option ..... 8

3.2.3  Atmospheric Drag Effects Option ..... 8

3.2.4  Gravitational and Drag Effects Option 9

3.3  Initial State Vector Option Menu ............. 9

3.3.1  Orbital Elements .................... 9

3.3.2  Initial Position and Velocity ...... 15

3.4  Time Control Parameters Menu ................ 18

3.5  Output Format Options Menu .................. 21

3.5.1  Geographic Coordinates ............. 21

3.5.2  Common Radar Coordinates ........... 21

3.5.3  Geospherical Inertial Coordinates .. 24

3.5.4  Earth-Centered-Inertial Coordinates  26

3.6  Menu Editing ..................................26

4  TRAJECTORY MAKER OUTPUT DESCRIPTION ................ 28

5  EXAMPLE ORBITS AND TRAJECTORIES .................... 31

5.1  Space Shuttle Discovery ..................... 31

5.2  Repeating Ground Trace ...................... 31

5.3  Sun-synchronous Orbit ....................... 32

5.4  Candidate Surveillance Orbits ............... 33

5.5  An Example Illustrating Orbit Decay ......... 34

5.6  Orbits Near Transition Region ............... 35

iv

5.7  Ballistic Missile Trajectories .............. 35

5.7.1  Western Test Range Trajectories .... 35

5.7.2  Hypothetical Threat Trajectories ... 35

6  TRAJECTORY SCAPE INPUT DESCRIPTION ................. 37

6.1  World Map Projections Options Menu .......... 37

6.1.1  Mercator with stereographic

projection ......................... 37

6.1.2  Hammer-Aitoff Equal Area Projection  38

6.1.3  Mollweide Equal Area Projection .... 38

6.2  Map Resolution Options Menu ................. 38

6.2.1  Fine Option ........................ 39

6.2.2  Medium Option ...................... 39

6.2.3  Coarse Option ...................... 39

6.3  World Map Focus Options Menu ................ 39

6.3.1  Greenwich Meridian ................. 39

6.3.2  American Region .................... 40

6.3.3  Soviet Region ...................... 40

7  TRAJECTORY SCAPE OUTPUT DESCRIPTION ................ 41

8  ADDITIONAL VIEWING CAPABILITY ...................... 45

8.1  Small Circles ............................... 45

8.2  Segmental Arcs and Spherical Polygons ....... 45

APPENDIX A.  HISTORICAL NOTES ........................ A-1

APPENDIX B.  GLOSSARY ................................ B-1

v

LIST OF MENUS

Menu                                                 Page

1  Unit Convention Menu ............................. 7

2  Perturbation Model Option Menu ................... 8

3  Initial State Vector Option Menu ................. 9

4  Initial Orbital Elements ........................ 10

5  Initial Position and Velocity ................... 15

6  Time Control Parameters ......................... 18

7  Output Format Option Menu ....................... 21

8  World Map Projections Menu ...................... 37

9  Map Resolution Options Menu ..................... 38

10  World Map Focus Menu ............................ 39

vi

LIST OF FIGURES

Figure                                                Page

1  Semimajor Axis and Eccentricity ................. 11

2  Orbit Inclination ............................... 13

3  Longitude of the Ascending Node and Argument of

Perigee ......................................... 14

4  Earth-Centered-Inertial Coordinates ............. 16

5  Geographic Coordinates .......................... 22

6  Common Radar Coordinates ........................ 23

7  Geospherical Inertial Coordinates ............... 25

8  Sample Trajectory Maker Graphics Display ........ 30

9  Four Tundra Orbits on Mercator and Stereographic

Projections ..................................... 42

10  Circular, 45 Degree Inclined Orbit on an

Hammer-Aitoff Projection ........................ 43

11  Circular, 45 Degree Inclined Orbit on a

Mollweide Projection ............................ 44

vii

WELCOME

Welcome  to  Trajectory Maker  and  its  companion program

Trajectory Scape.  Whether you  are an experienced orbital

mechanic,  relatively new  to the  field  or just  want to

learn about it, ORBIT HORIZONS  is confident that you will

enjoy using these tools.

Trajectory Maker  is a professional  simulation and design

tool  for  scientists and  engineers  involved  in mission

analysis and orbital operations.

TRAJECTORY MAKER FEATURES

Trajectory Maker predicts  trajectories for earth orbiting

satellites and ballistic missiles.  It produces high qual-

ity  trajectories  and  it can  accommodate  a  variety of

applications.

o  Trajectory  Maker has  four optional  force models:

Keplerian two-body motion,  gravity perturbations,

atmospheric drag perturbations and combined gravity

and atmospheric drag perturbations.

o  Graphics  displays  let  you  visualize  trajectory

ground traces,  as their  computed, simultaneously,

on three conformal map projections.  Altitude trend

is  also  displayed,  and there  is  a  window that

provides numerical time and altitude data.

o  Four optional  numeric output  formats are created:

geographic  coordinates, common  radar coordinates,

geospherical  inertial coordinates  and rectangular

Earth-centered-inertial  coordinates.   In addition

all input  data is echoed  to a  file with various,

auxiliary orbit parameters.

o  Flexibility  is  provided with  input  time control

options:  date  and   time  of  injection,  mission

duration  time,  granularity  of  computations  and

output sampling frequency.

o  Optional  initial  state  vector  input:  classical

orbital    elements    or   Earth-centered-inertial

coordinates.  Metric or English units may be used.

viii

Trajectory Maker's companion program, Trajectory Scape, is

a multiple  trajectory viewing tool.  It displays traject-

ories, created with Trajectory  Maker, on several optional

map  projections, e.g.,  Mercator,  stereographic, Hammer,

Mollweide, etc.  It has  three optional map resolutions as

well as three central viewing regions.

In  addition to  trajectory ephemerides,  Trajectory Scape

constructs  and  displays   circles,  segmental  arcs  and

spherical polygons on the various map projections.

Both  programs are  easy  to use  while  providing maximal

flexibility and  control for  your particular application.

They  comprise   an  excellent   introduction  to  orbital

mechanics,  and  are  outstanding  tools  for professional

scientists and engineers.

Twenty-four example data sets  are included in this User's

Guide for you to get started immediately.

The software requires an IBM  PC XT/AT or compatibles with

a  hard  drive  and  color  card.   Graphics visualization

requires EGA or VGA.  A  math co-processor is optional and

is utilized if present.

ix

Trajectory Maker

User's Guide                                            1

����������������������������������������������������������

1.  INTRODUCTION

According  to  the  Lomgman   Dictionary  of  Astronomy  &

Astronautics*,  a trajectory  is "the  path  of a  body in

space  or through  the  Earth's atmosphere.   The  word is

usually used to  describe the paths  of rockets and space-

craft rather than that of  planets or moons."  An orbit is

"the path of one body  in space around another."  Thus, an

orbit is a trajectory and  both terms will be used herein,

depending on the context.

Trajectory  Maker does  just what  its  name implies  - it

makes trajectories.  In  particular, it makes trajectories

for objects  under the  influence of  the Earth's gravita-

tional field, whose  mass is small  relative to the Earth.

Such  an object  may  be a  manmade  satellite continually

revolving around  the Earth,  it may  be an interplanetary

spacecraft having achieved escape velocity  or it may be a

ballistic missile on a destination towards its target.

In  any case,  the type  of  trajectory an  object attains

depends on the  energy manifested by  its initial position

and  velocity.   Trajectory   Maker  really  doesn't  care

because  it integrates  the  object's equations  of motion

directly,  rather  than   using  the  classical  universal

variables approach implied by two-body theory.

Because  the  shape  of   the  Earth  is  more  accurately

represented by  an ellipsoid  of revolution  than it  is a

sphere, trajectories deviate from idealized conic sections

implied by the universal variables.

Moreover, depending  on the objects  altitude, the Earth's

atmosphere tends to retard  the objects motion, causing it

to  decay, ultimately  colliding with  the Earth.   In the

case  of  ballistic  missile  trajectories,  the retarding

force becomes much greater than the gravitational force as

the object rips through the atmosphere towards its target,

and if its mission is aborted, atmospheric winds may cause

its fragments to drift.

����������������������

*Ridpath, Ian, Longman Illustrated Dictionary of Astronomy

&  Astronautics,  Longman Group  UK  Limited,  Burnt Mill,

Harlow, Essex, 1988.

Trajectory Maker

User's Guide                                            2

����������������������������������������������������������

By  integrating the  equations  of motion  directly, these

perturbation   effects   may  be   treated   without  much

difficulty.

Specifically, Trajectory Maker's force model includes both

atmospheric  drag and  gravitational  perturbation models.

The differential  equations of motion  are integrated with

the Runge-Kutta-Shank's eighth-order, fixed-step numerical

scheme  having  twelve  stages.   Its  speed  is moderate,

depending on the user selected step-size, but it is stable

and very accurate.

Trajectory maker also contains an innovative algorithm for

rapid, high precision altitude computations.  All computa-

tions are performed with double precision arithmetic.

However, you don't  need to know or  be concerned with the

technical details to  use the program  - that's one reason

for the program !

For the  interested reader, some  historical background is

given in  Appendix A.   A glossary  of terms  used in this

guide is given in Appendix B.

Trajectory Maker

User's Guide                                            3

����������������������������������������������������������

2.  INSTALLATION

"Trajectory Maker" and "Trajectory Scape" are designed for

use on  IBM PC  XT/AT and  compatibles with  a hard drive.

Numeric  data is  available on  any  monitor with  a color

card.  Graphics visualization requires an EGA or VGA card.

A math co-processor is optional and used if present.

Make  a sub-directory  and copy  all of  the files  on the

distribution disk(s) from your floppy drive.  For example,

C:>MD TMAKER

C:>CD TMAKER

Insert the distribution disk(s) in Drive A: then,

C:>COPY A:*.*

That's it for installation.  You should have the following

files in your subdirectory:

TMAKER.EXE ... Trajectory Maker Executable Program.

TSCAPE.EXE ... Trajectory Scape Executable Program.

TMSRB.FON .... Graphics Fonts.

WORLDB.LGE ... Large World Map.

WORLDB.MED ... Medium World Map.

WORLDB.SML ... Small World Map.

Running Trajectory Maker

������������������������

Before giving a detailed  description of the input options

and data, let's go ahead and run an example to familiarize

you with the operation of the program.

Type TMAKER at the DOS  prompt.  After the opening banner,

a  menu  will  appear  requesting  your  choice  of units.

Select the Nautical  Miles option. (You  may abort the job

by pressing the Ctrl-Z sequence.)

* Type 1 and press <Enter>

Next,  a menu  appears requesting  your choice  of models.

Select the Unperturbed two-body model.

* Type 1 and press <Enter>

Trajectory Maker

User's Guide                                            4

����������������������������������������������������������

Next, a menu  appears requesting your  choice of input for

the  initial  conditions.   Select  the  Orbital  Elements

Option.

* Type 1 and press <Enter>

The  next  menu  requests  the  orbital  elements  of  the

satellite.   These elements  characterize the  orbit.  For

this example,  we will  assume the  satellite will  have a

circular orbit at an altitude  of 400 nautical miles (nm),

in  a  plane  inclined  45  degrees  with  respect  to the

equatorial plane.

The radius of the Earth is  3444 nm at the equator, so the

semimajor axis is approximately: a = 3844 nm.

* Type 3844 and press <Enter>

Since the orbit is circular, its eccentricity, e = 0.

* Type 0 and press <Enter>

The satellite's orbit plane has inclination, i = 45�.

* Type 45 and press <Enter>

The  next two  entries  are orientation  angles  which are

described in Section 3.3.1.  Assume � = 0�, and w = 0�.

* Type 0 and press <Enter>

* Type 0 and press <Enter>

Notice  that  the  period  of  the  satellite,  about 6000

seconds, is  displayed.  The period  is the  time it takes

the satellite to make one  revolution about the Earth.  It

is displayed to  aid you in  selecting the next parameter,

the time past  perigee, when you wish  to start the orbit.

We will use � = 0 seconds.

* Type 0 and press <Enter>

That completes the orbit description.

The next  menu is used  to define the  time parameters for

the orbit.  Assume the duration of the mission is one day,

which is about 86400 seconds,  starting at time 0.  Since,

for this example  we shall not be  concerned with the date

Trajectory Maker

User's Guide                                            5

����������������������������������������������������������

and time  of the  orbit injection,  enter 0  or just press

<Enter> in the  date and time fields.   When the cursor is

in the Mission Duration Time field,

* Type 86400 and press <Enter>

The  next   parameter  is   the  orbit   propagation  time

increment.  We will use one minute, or 60 seconds.

* Type 60 and press <Enter>

The final time parameter  is the output sampling frequency

for the data file.  We will use the same value as the time

increment.

* Type 60 and press <Enter>

The next menu requests your  choice of output formats.  We

will select  the geographic or  geodetic coordinates which

are the most common.

* Type 1 and press <Enter>

You are asked if you want  to view the trajectory.  We do,

so answer yes.

* Type y and press <Enter>

The  program  begins  by  displaying  the  world  on three

projections which  are described  in Section  4.  When the

maps are  drawn, you  will be prompted  to press  a key to

start the orbit computations.

When the orbit propagation is complete, your screen should

have a figure  resembling Figure 8 in  Section 4.  You may

select  a black  and white  palette if  you want  a screen

dump.  Then you may press any key to end.

Section 5 contains several examples,  for you to try, that

illustrate the various trajectories  that you can generate

with Trajectory Maker.

Running Trajectory Scape

������������������������

To run  Trajectory Scape, type  TSCAPE at  the DOS prompt.

After the opening  banner, a menu  appears requesting your

Trajectory Maker

User's Guide                                            6

����������������������������������������������������������

choice of world map projections.

* Type 2 and press <Enter>

Next,  a  menu  appears  requesting  your  choice  of  map

resolution.

* Type 3 and press <Enter>

Next, a menu  appears requesting your  choice of world map

focus.

* Type 2 and press <Enter>

You will be prompted, in  the command window, for an orbit

file  name.   The output  file  that was  just  created by

Trajectory Maker is HOST.DAT.  To display this file,

* Type HOST.DAT and press <Enter>

Your screen should  have a figure  resembling Figure 10 in

Section 7.   You may select  a black and  white palette if

you want  a screen  dump.  Then you  may press  any key to

end.

The next section describes the input parameters in detail.

Trajectory Maker

User's Guide                                            7

����������������������������������������������������������

3.  TRAJECTORY MAKER INPUT DESCRIPTION

Trajectory  Maker is  very easy  to use.   Just go  to the

subdirectory  where you  installed  the programs  and type

TMAKER at  the DOS  command line.   You will  be presented

with a  series of five  principle menus.   These menus and

their  input parameters  are described  in detail  in this

section.

3.1  Unit Convention Menu

You are given  the option of working  in either English or

metric units.  In either case all time units are processed

in seconds.  Both input and output for angles are measured

in decimal degrees.

If  you  choose  the  English  Units  Option,  then length

measurements must be input in nautical miles, and velocity

measurements in nautical miles per second.

If  you  choose  the  Metric  Units  Option,  then  length

measurements  must be  input  in kilometers,  and velocity

measurements in kilometers per second.

These options are illustrated in Menu 1 below.

����������������������������������������������Ŀ

�               Unit Convention                �

�                                              �

�    1.  English   ( nm, lb, sec )             �

�    2.  Metric    ( km, kg, sec )             �

�                                              �

�    Type option number and press Enter: 1     �

������������������������������������������������

Menu 1.  Unit Convention.

3.2  Perturbation Model Option Menu

You are given four force  model options for tailoring your

particular application.  These  options are illustrated in

Menu 2 below.

Trajectory Maker

User's Guide                                            8

����������������������������������������������������������

����������������������������������������������Ŀ

�          Perturbation Model Option           �

�                                              �

�    1.  Unperturbed two-body                  �

�    2.  Gravitational harmonic effects        �

�    3.  Atmospheric drag effects              �

�    4.  Gravitational and atmospheric effects �

�                                              �

�    Type option number and press Enter: 1     �

������������������������������������������������

Menu 2.  Perturbation Model Option.

3.2.1   Unperturbed Two-body Option.

This  option assumes  that  the Earth's  gravity  field is

modeled with an homogeneous sphere.  The orbit motion will

be along a  conic section (ellipse,  parabola or hyperbola

depending on  the initial  conditions).  However, altitude

is  based  on the  Earth  having  the shape  of  an oblate

spheroid.  The altitude of a circular orbit will therefore

not normally be a constant.

3.2.2  Gravitational Harmonic Effects Option.

This  option assumes  that  the Earth's  gravity  field is

modeled with an  oblate spheroid (A  surface of revolution

generated by rotating an ellipse about its minor axis).

If you choose this option, you  will be asked to specify a

positive number between 2  and 22 (inclusive).  The larger

the  number, the  more  precise the  gravity  model. (This

number corresponds to  the order of  the gravity potential

function.)  For  example, use order  2 for Sun-synchronous

orbits.

3.2.3  Atmospheric Drag Effects Option.

This option takes  into account the  drag force created by

the Earth's atmosphere.   This force varies  with the mass

of the object, its  effective cross-sectional area and its

drag  coefficient.   These three  parameters  are commonly

lumped together  into a  single parameter,  which you will

ask to supply,  if you choose  this option. This parameter

Trajectory Maker

User's Guide                                            9

����������������������������������������������������������

is called the ballistic coefficient, �, and conveniently

has units measured in lbs/sq-ft,  or kg/sq-m, sort of like

density.

Heavy masses  tend to have  larger �  than lighter masses,

but large areas have smaller  � than small areas.  A space

based radar antenna may have  a � of 0.5 lb/sq-ft, whereas

a reentry  vehicle may  have a  � varying  from 5  to 1000

lb/sq-ft, for  example. The definition  used is consistent

with that used at the Western Test Range.

3.3.4  Gravitational and Atmospheric Drag Effects Option.

You may choose  both gravity and  drag perturbations.  For

reentry  trajectories,  you should  consider  this option,

with the gravity model having Order 2 or 4.

3.3  Initial State Vector Option Menu

You  may input  the  initial state  vector  in one  of two

forms.  Either with  Earth-Centered-Inertial (ECI) coordi-

nates or classical orbital elements.

These options are illustrated in Menu 3 below.

����������������������������������������������Ŀ

�         Initial State Vector Option          �

�                                              �

�    1.  Orbital Elements                      �

�    2.  Earth Centered Inertial               �

�                                              �

�    Type option number and press Enter: 1     �

������������������������������������������������

Menu 3.  Initial State Vector Option.

3.3.1  Orbital Elements

Orbital  elements  are  classical,  fundamental parameters

that characterize  an orbit.   They are  defined in almost

any elementary  book on astronomy,  celestial mechanics or

astrodynamics.  The input format  for the orbital elements

is illustrated in Menu 4 below.

Trajectory Maker

User's Guide                                            10

����������������������������������������������������������

����������������������������������������������Ŀ

�          Initial Orbital Elements            �

�                                              �

� a  3844   nm     e  0         i  45      deg �

�                                              �

� �  0             w  0         �  0       sec �

������������������������������������������������

Menu 4.  Initial Orbital Elements.

a - Semimajor axis

������������������

This element defines the size of the orbit.  The semimajor

axis of an ellipse or hyperbola is the distance from their

centers to their respective vertices.  As used herein, the

semimajor  axis of  a parabola  is  the distance  from its

focus to its  vertex.  A circular orbit  is a special case

of an ellipse.  Its semimajor axis is, simply, its radius.

Input this parameter with the  units selected in the Units

Option Menu.  This element is illustrated in Figure 1.

e - Eccentricity

����������������

This  element defines  the shape  of the  orbit.  It  is a

fundamental parameter in the definition of conic sections.

It is dimensionless and can be any nonnegative number.  If

it is zero, the  orbit is circular; if  it is between zero

and one, the orbit  is an ellipse; if  it is one the orbit

is a parabola; and if it  is greater than one the orbit is

an hyperbola.   These configurations  are also illustrated

in Figure 1.

Remark:  If  the orbit is  an ellipse and  the minimum and

maximum distance from  the Earth's center  is known (i.e.,

its perigee and apogee radius) then its semimajor axis is

given by their mean and  the eccentricity is their differ-

ence divided by their sum.

i - Inclination

���������������

This element defines the orientation of the orbit plane

Trajectory Maker

User's Guide                                            11

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 1                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            12

����������������������������������������������������������

relative to the equatorial plane.  It may be in the range,

0  � i  � 180  degrees.  If  i �  90 degrees,  the maximum

latitude of the orbit's ground trace will be equivalent to

i, otherwise the maximum  latitude will be its complement,

90  - i.   Equatorial  orbits have  zero  inclination, and

polar orbits have 90  degree inclination.  This element is

illustrated in Figure 2.

� - Longitude of the ascending node

�����������������������������������

This element  defines the  orientation of  the orbit plane

relative to the vernal equinox or, optionally, the initial

Greenwich meridian  (see Section  3.3.2).  Inclined orbits

intersect the equatorial  plane in two  points.  The point

where the orbiting object  intersects the equatorial plane

on  its  journey  from  south  to  north  is  the  orbit's

ascending node. The other point is its descending node.

The line  connecting the two  nodes is the  line of nodes.

The longitude of the ascending node is the angle, measured

in the equatorial plane, between the vernal equinox or the

initial Greenwich meridian  and the line  of nodes. It may

be in  the range, 0  � �  < 360 degrees.   This element is

illustrated in Figure 3.

w - Argument of perigee

�����������������������

This  element  defines  the  orientation  of  the object's

perigee in the orbit plane relative to the ascending node.

It is  the angle measured  in the orbit  plane between the

line of  nodes and the  line connecting the  center of the

Earth and  the object's radius  of perigee.  It  may be in

the range,  0 �  w <  360 degrees.   This element  is also

illustrated in Figure 3.

� - Time past perigee

���������������������

This element defines the object's initial location in the

orbit plane  relative to the  argument of  perigee.  It is

measured in seconds, and for elliptical orbits is normally

not greater  than the  orbital period.   If the  orbit is,

indeed, an ellipse, the program computes the orbital

Trajectory Maker

User's Guide                                            13

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 2                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            14

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 3                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            15

����������������������������������������������������������

period and displays the value on your screen.  This is

useful  in making  a choice  regarding  when to  start the

orbit,  and in  the  Time Options  Menu,  when to  end the

orbit.

3.3.2  Initial Position and Velocity

Rectangular, Earth-Centered-Inertial (ECI) coordinates are

the fundamental  coordinates used in  Trajectory Maker for

measuring  an  objects  motion,  i.e.,  its  position  and

velocity.  Indeed,  the equations  that govern  the motion

are propagated relative  to this reference  frame.  If the

orbital elements option is  selected, they are transformed

into ECI coordinates prior  to propagating the orbit.  The

input format for these  coordinates is illustrated in Menu

5 below.

����������������������������������������������Ŀ

�        Initial Position and Velocity         �

�                                              �

� x 1404.72   y 845.78    z 3242.26   (nm)     �

�                                              �

� xd -0.959   yd -2.244   zd 2.436    (nm/sec) �

������������������������������������������������

Menu 5.  Initial Position and Velocity.

There are two options for  the ECI coordinate system.  One

option  assumes  that  the principle  x-axis  lies  in the

equatorial plane  and is  in the  direction of  the vernal

equinox, as illustrated in Figure 4.

The other option assumes that the principle x-axis lies in

the  equatorial  plane  and is  in  the  direction  of the

Greenwich  meridian at  the  time of  orbit  insertion and

remains fixed in that direction  in space for the duration

of  the trajectory.   The  Greenwich meridian,  of course,

rotates relative to this frame.  This option is useful for

positioning satellites for geographical viewing regions.

Your  selection  may  be obtained  with  the  Time Control

Parameters menu described in Section 3.4 below.

The ECI coordinate definitions are as follows:

Trajectory Maker

User's Guide                                            16

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 4                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            17

����������������������������������������������������������

x - coordinate

��������������

Position coordinate  in the equatorial  plane, positive in

the  direction  of  the  vernal  equinox  or,  optionally,

initially  aligned  in  the  direction  of  the  Greenwich

meridian.

y - coordinate

��������������

Position coordinate in the  equatorial plane orthogonal to

the x axis,  and directed 90  degrees to the  east so that

the system is right-handed.

z - coordinate

��������������

Position coordinate coincident with the Earth's spin axis,

positive northwards.

xd - velocity component

�����������������������

Velocity component in the x-direction.

yd - velocity component

�����������������������

Velocity component in the y-direction.

zd - velocity component

�����������������������

Velocity component in the z-direction.

Once the initial state vector  has been entered, a message

is displayed relating  the period of  the orbit, an escape

condition or  the Kepler flight  time (no  drag or oblate-

ness).

This latter condition  occurs if the  trajectory is likely

Trajectory Maker

User's Guide                                            18

����������������������������������������������������������

to impact with  the Earth.  These  parameters are provided

to aid you in your choice of the time parameters below.

3.4  Time Control Parameters

These parameters  control the  time span  and precision of

the trajectory.  The input  format for these parameters is

illustrated in Menu 6.

����������������������������������������������Ŀ

�           Time Control Parameters            �

�                                              �

�  Date and time of injection:                 �

�    Year 1992    Month 2        Day 3         �

�    Hours 22     Minutes 27     Seconds 30    �

�                                              �

�  Mission duration time 86400       Seconds   �

�  Propagation increment 300         Seconds   �

�  Output frequency 60               Seconds   �

������������������������������������������������

Menu 6.  Time Control Parameters.

The  date and  time of  injection are  used to  define the

initial  Greenwich sidereal  time  when the  orbit begins.

The  time must  be input  in  Universal Time  units (i.e.,

Greenwich Mean Time, or Zulu Time).

If these data are entered,  the Greenwich sidereal time is

computed and  the initial  state vector  is assumed  to be

referenced relative to the vernal equinox.

If  these  data  are not  entered,  the  initial Greenwich

sidereal  time is  set  to zero  and  the state  vector is

assumed  to  be  referenced   relative  to  the  Greenwich

meridian.

Year

����

Year the  orbit begins.   This item  must be  a four digit

number such as 1992.

If you enter zero  or hit the enter  key, the cursor skips

Trajectory Maker

User's Guide                                            19

����������������������������������������������������������

to the hours'  field, and the  inertial reference frame is

assumed to be  the aligned with  the Greenwich meridian at

the  initial time,  t0, defined  with the  hours, minutes,

seconds data fields.

Month

�����

Month the orbit  begins.  This item must  be a number with

no more than two digits, such as 1 or 12.

Day

���

Day the orbit begins.  This item  must be a number with no

more than two digits, such as 1 or 31.

Hours

�����

Hours past midnight when the orbit begins.  This item must

be a number with  no more than two  digits such as 9 which

means 9:00 am, or 15 which means 3:00 pm.

Minutes

�������

Minutes past  the hour when  the orbit  begins.  This item

must be a number with no more than two digits such as 3 or

15.

Seconds

�������

Seconds  past minutes  when the  orbit begins.   This item

must be a number with no more than two digits such as 3 or

30.

Mission duration time  (tf)

���������������������������

This parameter  defines the  time span  of the  orbit, and

Trajectory Maker

User's Guide                                            20

����������������������������������������������������������

must be input in seconds.

If your  application is a  reentry trajectory  and you are

using  a  perturbation  model,  the  input  value  for the

mission duration should  be larger than  the Kepler flight

time.  The Kepler flight time is  shown to aid you in your

selection.

Propagation increment  (dt)

���������������������������

This parameter is the  propagation step-size (seconds). It

controls the precision of  the computations, and should be

a multiple of the duration time above, and must also be in

seconds.

Output frequency  (pt)

����������������������

You may want to propagate the trajectory at one step-size,

dt,  to maintain  accuracy, but  output  the data  at some

larger multiple of dt.  If  you just press the return key,

with  no  data  entry,   the  program  will  default  this

parameter to the propagation increment above.

Numerical experience has shown that for non-drag perturbed

trajectories,  a  step size  of  300 sec  will  give seven

significant figures accuracy for tf  = 86400 sec (i.e. one

day),  six figures  for 14  days and  five figures  for 28

days.  This  latter case means  that your  results will be

accurate  to five  decimal  digits after  8064 integration

steps.

To maintain stability and  numerical integrity for reentry

trajectories, you will  have to cut  the step-size down to

10, or even 5, seconds, depending on the ballistic coeffi-

cient.  You should  not have to go  below 1 second, unless

you  are dealing  with  very light  masses  with ballistic

coefficients of the order of 1 lb/sq-ft or less.

Program execution time will  vary depending on your choice

of the  above time  parameters.  On a  12 MHz  AT having a

math co-processor, for unperturbed  orbits, assume 77 msec

per step, add  20 msec for each  gravity harmonic, and add

62 msec per step for drag effects.

Trajectory Maker

User's Guide                                            21

����������������������������������������������������������

3.5  Output Format Options Menu

You may choose one of  four output format options from the

menu illustrated in Menu 7.

����������������������������������������������Ŀ

�            Output Format Option              �

�                                              �

�    1.  Geographic coordinates                �

�    2.  Common radar coordinates              �

�    3.  Geospherical inertial coordinates     �

�    4.  Earth centered inertial coordinates   �

�                                              �

�    Type option number and press Enter: _     �

������������������������������������������������

Menu 7.  Output Format Option.

3.5.1  Geographic Coordinates.

These coordinates are standard geographic coordinates that

are  used  for  maps:  latitude,  longitude  and altitude.

Geodetic altitude, h, and  latitude, �, are illustrated in

Figure 5.

The  altitude of  an  object is  its  instantaneous height

above the Earth's surface.  Since  the Earth is modeled as

an oblate spheroid, the  polar and equatorial radii differ

by about 10 nautical  miles, thus, circular orbits usually

will not have constant altitude.

Latitude is the instantaneous angle between the equatorial

plane, and the line that contains the object and is normal

to the Earth's surface.

Longitude is the instantaneous angle between the Greenwich

meridian  and the  projection of  the line  connecting the

object with the Earth's center.

3.5.2  Common Radar Coordinates.

These coordinates,  illustrated in Figure  6, are commonly

used for measurements at radar sites.  They consist of the

object's range, azimuth and elevation relative to the

Trajectory Maker

User's Guide                                            22

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 5                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            23

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 6                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            24

����������������������������������������������������������

radar site coordinates.

Range, R, is the distance from the site to the object.

Azimuth, A, is the angle  between the object and the radar

site's meridian, measured clockwise from the north.

Elevation, E, is the angle above or below the horizon.

.

Range rate, R, is the rate of change of range with respect

to time.

If you select  this option, a  menu appears requesting the

geographic  coordinates of  the  radar site.   Namely, its

geodetic latitude, longitude and altitude.  It is tempting

to input altitude in feet or meters, but it must be in the

same units selected in the Unit Options Menu.

3.5.3  Geospherical Inertial Coordinates.

This output format contains  the magnitude of the object's

radius vector,  its right  ascension and  declination.  It

also  contains its  inertial velocity  magnitude, velocity

azimuth  and  flight-path  angle.   These  parameters  are

illustrated in Figure 7.

The magnitude of the radius  vector is, r, is the distance

from the Earth's center to the orbiting object.

As used herein,  the right ascension of  the object is the

angle,  �, measured  in the  equatorial plane  between the

vernal equinox or  the initial Greenwich  meridian and the

projection of the radius vector onto the equatorial plane.

The declination of the object, �, is the angle between the

equatorial plane and the  radius vector.  It is equivalent

to geocentric latitude.

The magnitude of  the velocity vector, v,  is the speed of

the object.

The  velocity  azimuth  angle, �,  is  the  angle measured

clockwise from the northern direction to the projection of

the velocity vector  on the local  horizontal plane.  (The

local horizontal plane  is the plane  normal to the radius

vector.)

Trajectory Maker

User's Guide                                            25

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 7                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            26

����������������������������������������������������������

The  flight-path  angle,  �,  is  the  angle  between  the

velocity vector and the local horizontal plane.

3.5.4  Earth-Centered-Inertial Coordinates.

This  output  format  contains  the  object's rectangular,

Earth-centered-inertial  position,   velocity  and  accel-

eration components in double precision.  Their definitions

are given in Section 3.3.

Once you have made your  choice for output format, you are

given the  choice of either  text or  graphics output.  If

you just want  the data file, choose  the text format.  If

you choose  graphics format,  you will  get both  text and

graphics output.   The graphics  display is  in geographic

format regardless of your choice of output format.

3.6  Menu Editing

You may change the value of  a data item in any data entry

field  without exiting  the program  with  the Tab  key or

Shift-Tab key sequence.

Pressing the  Tab key  moves the  cursor to  the next data

field.  Pressing the  Shift-Tab sequence (or  the Esc key)

moves the cursor to the previous field.

For example, if you  entered a data item  that you wish to

change, simply  move to the  field that  contains the data

item, reenter  the new number  and press  <Enter>.  Do not

try to edit the data item with the Arrow, Ins or Del keys.

The data in a  data field must be  reentered.  You can, of

course, use the Backspace key.

If you move to a field that contains data that you wish to

keep, skip over it with the  Tab key or Shift-Tab key.  Do

not press <Enter> if you want to keep a previously entered

data item, as this has the effect of setting the data item

to zero.  Press <Enter> only when you enter a data item or

you want a data item to have the value zero.

You may  return to  a previous menu  by using  the Esc key

whenever your  cursor is  in the  first data  field of the

current menu.  If you press this key when the current menu

is the  Units Option  Menu, the  job is  aborted.  You can

always abort a job with the Ctrl-Z sequence.

Trajectory Maker

User's Guide                                            27

����������������������������������������������������������

If you are satisfied with your entries you can move to the

next menu with the Tab key.

Trajectory Maker

User's Guide                                            28

����������������������������������������������������������

4.  TRAJECTORY MAKER OUTPUT DESCRIPTION

The output data is in ASCII format and is contained in two

files.  For  convenience, one file,  HOST.HED, echoes your

input  data.   The  other  file,  HOST.DAT,  contains your

selected  output  format from  the  Output  Format Options

menu.  You may  wish to rename these  files, and save them

for use in the companion program, Trajectory Scape.

The first record in  HOST.DAT contains information used by

Trajectory Scape.   Just delete  this record,  and you can

plot any  desired data  items with  your favorite plotting

package.

If you selected  Geographic Coordinates, HOST.DAT contains

four data columns: time,  geodetic latitude, longitude and

altitude, in  that order.  These  parameters are described

in Section 3.5.1.

If  you   selected  Common   Radar  Coordinates,  HOST.DAT

contains   five  data   columns:  time,   range,  azimuth,

elevation and range rate, in that order.  These parameters

are described in Section 3.5.2.

If   you  selected   Geospherical   Inertial  Coordinates,

HOST.DAT contains seven data  columns: time, radius, right

ascension, declination, speed, velocity azimuth and flight

path angle, in that order.  These parameters are described

in Section 3.5.3.

If   you  selected   Earth-Centered-Inertial  coordinates,

HOST.DAT contains ten data  columns: time, three position,

three velocity and three  acceleration components, in that

order.   The  three  components  represent  the  x,  y,  z

directions.   These  parameters are  described  in Section

3.3.2.

In addition  to the numeric  data, you have  the option of

displaying the  trajectory's ground trace  on your monitor

while it is being  computed.  The graphics format displays

three conformal views of the trajectory.

(1)  A Mercator projection.

(2)  A stereographic projection of the northern

hemisphere.

Trajectory Maker

User's Guide                                            29

����������������������������������������������������������

(3)  A stereographic projection of the southern

hemisphere.

Conformal  projections  are   useful  because  angles  are

preserved  under  the  mapping.   Therefore,  the relative

shapes of objects are preserved.  Other projections may be

viewed with Trajectory Scape (see Section 6).  A sample of

the graphics display is shown in Figure 8.

At the top  of the screen  there is a  chart that displays

the altitude  trend as  it is  being computed.   This is a

very  useful display,  for  you can  immediately correlate

altitude with location.

The actual numeric altitude values that are being computed

are  displayed  in a  text  window  at the  bottom  of the

screen.  Time  is also displayed  in this  window in sexa-

gesimal format, that is, days:hrs:mins:secs:

You  may  pause the  visual  display by  pressing  any key

except the  Esc or the  Ctrl-Z sequence.   Either of these

keys will abort the job and return the system to DOS.

When the computations  have been completed  you may change

the  color palette  to black  and  white.  This  option is

included so that a readable screen dump may be obtained on

a  black  and  white printer.   It  should  be  noted that

standard  screen  dumps  give a  distorted  image  on some

printers.  However, there  are several commercial products

on the market that  will provide color and distortion-free

prints.

Trajectory Maker

User's Guide                                            30

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 8                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            31

����������������������������������������������������������

5.  EXAMPLE ORBITS AND TRAJECTORIES

This  section  provides  initial  conditions  for  various

trajectories that Trajectory Maker  is capable of predict-

ing.

5.1 Space Shuttle Discovery Mission STS-42

The  space shuttle  Discovery  was launched  on Wednesday,

January  22,  1992.    It  was  inserted   into  orbit  at

18:52:32.960  Greenwich  Mean   Time  with  the  following

orbital elements, obtained  from L. B.  J. Space Center in

Houston, Texas.

a = 3603.1925 nautical miles (nm)

e = 0.000626

i =  56.74595�

� = 242.95604�

w =  65.48579�

� = 2781.72794 sec

The following drag related parameters were also given:

Weight = 231448.0 lb

Drag Coefficient = 2.0

Effective drag area = 2750.0 ft�

The ballistic coefficient is, therefore,

� = 231448/(2 X 2750) = 42.08 lb/ft�

Try  a  gravity model  of  Order 2  with  atmospheric drag

effects  (Model  4)  with  the  above  parameters  and the

following time parameters:

Year: 1992    Month: 1      Day: 22

Hour: 18      Minutes: 52   Seconds: 32.96

Mission duration (tf): 86160 sec (one sidereal day)

Propagation increment (dt): 60 sec

Output frequency (pt): 60 sec

5.2  Repeating Ground Trace

If a satellite's period is a multiple of the time it takes

for the Earth to make a  complete rotation on its axis (23

Trajectory Maker

User's Guide                                            32

����������������������������������������������������������

hrs  56 min  or 86160  seconds),  then the  satellite will

retrace the same path over the Earth as it did initially.

The  following  parameters define  an  orbit  whose ground

trace repeats after 15 revolutions.  The period is 5743.84

seconds.

a = 3743 nm

e = 0

i = 60�

� = 0�

w = 0�

� = 0 sec

Try  an unperturbed  gravity  model (Model  1)  with these

elements and the following time parameters:

t0 = 0 sec

tf = 172320 sec (two sidereal days)

dt = 60 sec

pt = 60 sec

Repeat this  case with gravity  model of  order two (Model

2).  The ground trace will be displaced a few degrees from

the unperturbed case,  demonstrating the effect  of a non-

spherical Earth.

5.3  Sun-synchronous Orbit

An orbit in which a satellite passes over the same part of

the Earth at  the same time each  day is a Sun-synchronous

orbit.  The orientation  of  the orbit  plane  will remain

nearly fixed relative to the Sun as the Earth moves in its

orbit.

Such  an  orbit  is  achieved  by  adjusting  its  orbital

elements so that the regression  rate of the line of nodes

is just  that of the  Earth's angular rate  about the Sun,

which is about 1� each day (0.9856 deg/day).

The above orbit becomes Sun-synchronous if the inclination

is elevated from  60� to 97.61�.   The regression rate may

be inspected in the output file, HOST.HED.  Also note that

the orbit is a retrograde orbit.

Trajectory Maker

User's Guide                                            33

����������������������������������������������������������

5.4  Candidate Surveillance Orbits

The  next few  examples  generate repeating  ground traces

that  are  especially  good  candidates  for  surveillance

and/or communication.   The orbital  elements are adjusted

such  that apogee  is in  the northern  hemisphere,  where

they will dwell the longest.

Their  inclination is  critical  (63.44�) so  that gravity

will not  cause their line  of apsides to  rotate (so that

apogee will stay put).  This can be verified by inspecting

the file HOST.HED.  A value  of zero, or nearly so, should

be given for the argument of perigee rate.

Also, the  longitude of the  ascending node  and time past

perigee can be adjusted  to synchronize multiple orbits of

these types for various satellite constellation considera-

tions. For example, you could try � = 0�, 90�, 180�, 270�.

Tundra Orbits (24 hour period)

������������������������������

a =  22766         22766         22766         22766  nm

e =  0.33          0.69          0.69          0.69

i =  63.442�       63.44�        63.44�        63.44�

� =  0.00�         0.00�         0.00�         0.00�

w =  270.00�       270.00�       290.00�       300.00�

� =  0             0             0             0      sec

Model 1: Unperturbed Gravity Model

t0 = 0,   tf = 86160,   dt = 300,   pt = 300   (sec)

Molniya Orbits and Hookers (12 hour period)

�������������������������������������������

a =  14342         14342         14342         14342  nm

e =  0.69          0.69          0.69          0.69

i =  63.44�        63.44�        63.44�        63.44�

� =  0.0�          0.0�          0.0�          0.0�

w =  270�          90�         � 290�        � 310�  (etc)

� =  0.0           0.0           0.0           0.0    sec

Model 1: Unperturbed Gravity Model

t0 = 0,  tf = 86160,   dt = 120,   pt = 120   (sec)

Trajectory Maker

User's Guide                                            34

����������������������������������������������������������

An Eight-hour orbit

�������������������

a = 10945  nm

e = 0.594

i = 63.44�

� = 0�

w = 270�

� = 0  sec

Model 1: Unperturbed Gravity Model

t0 = 0,  tf = 86160,   dt = 120   pt = 120   (sec)

5.5  An Example Illustrating Orbit Decay

a = 6478 km

e = 0

i = 45�

� = 0�

w = 0�

� = 0  sec

Model 3: Drag perturbation model with � = 100 kg/m�

t0 = 0,   tf = 2000,   dt= 5,   pt= 5   (sec)

5.6  Orbits Near Transition Region

The next three  examples illustrate orbits  all having the

same radius of  perigee near the  transition region from a

closed to an open orbit.

Elliptical Orbit     Parabolic Orbit     Hyperbolic Orbit

����������������     ���������������     ����������������

a = 40000 nm             4000 nm             40000 nm

e = 0.9                  1.0                 1.1

i = 80�                  80�                 80�

� = 180�                 180�                180�

w = 0�                   0�                  0�

� = 0    sec             0    sec            0    sec

Model 1: Unperturbed Gravity Model

t0 = 0,  tf = 200660,   dt = 120   pt = 120   (sec)

Trajectory Maker

User's Guide                                            35

����������������������������������������������������������

5.7  Ballistic Missile Trajectories

The remaining set of trajectories are defined with initial

state  vectors  given in  ECI  coordinates  (Initial State

Vector Option 2). These  trajectories exit and reenter the

Earth's atmosphere on a predefined target destination.

5.7.1  Western Test Range Examples

These  two  hypothetical  trajectories  are  launched from

Vandenberg  Air   Force  Base  with   destination  in  the

Kwajalein Atoll.

X = -1705.16 nm

Y = -2509.17 nm

Z =  1926.63 nm

XD=    -3.102 nm/sec

YD=    -0.525 nm/sec

ZD=    -0.483 nm/sec

Model 4: Gravity model order 2 and � = 25 #/sq-ft.

t0 = 200,   tf = 2000,   dt = 5,   pt = 5   (sec)

X = -2525.03  nm

Y = -2146.25  nm

Z =  2012.86  nm

XD=    -2.608 nm/sec

YD=     1.351 nm/sec

ZD=    -0.318 nm/sec

Model 4: Gravity model order 2 with � = 75 #/sq-ft.

t0 = 475,   tf = 2000,   dt = 5,   pt = 5   (sec)

5.7.2  Hypothetical Threat Trajectories

The  next  three   trajectories  are  hypothetical  Soviet

launched ICBMs with Continental USA destination targets.

Trajectory Maker

User's Guide                                            36

����������������������������������������������������������

A Nominal Trajectory (Reentry angle 24 degrees)

�����������������������������������������������

X  =  1404.72 nm

Y  =   845.78 nm

Z  =  3242.26 nm

XD =    -0.959 nm/sec

YD =    -2.244 nm/sec

ZD =     2.436 nm/sec

Model 4: Gravity model order 2 and � = 1200 #/sq-ft.

t0 = 0,   tf = 1600,  dt = 10,  pt = 10   (sec)

A Depressed Trajectory (Reentry angle 20.17 degrees)

����������������������������������������������������

X  =  1149.67 nm

Y  =  1075.78 nm

Z  =  3348.70 nm

XD =     1.007 nm/sec

YD =    -2.889 nm/sec

ZD =     1.719 nm/sec

Model 4: Gravity model order 2 and � = 1200 #/sq-ft.

t0 = 0,  tf = 1600,   dt = 10,   pt = 10   (sec)

A Lofted Trajectory (Reentry angle 34.91 degrees)

�������������������������������������������������

X  =   980.98 nm

Y  =  1218.02 nm

Z  =  3523.47 nm

XD =    -0.393 nm/sec

YD =    -1.939 nm/sec

ZD =     2.787 nm/sec

Model 4: Gravity model order 2 and � = 1200 #/sq-ft.

t0 = 0,  tf = 2400,   dt = 5,   pt = 5   (sec)

Trajectory Maker

User's Guide                                            37

����������������������������������������������������������

6.  TRAJECTORY SCAPE INPUT DESCRIPTION

Trajectory Scape  is a  multiple trajectory  viewing tool.

Once you have created one  or more geographic output files

with trajectory Maker (Output option  1), you may view any

or all of  them with Trajectory Scape.   At the DOS prompt

type TSCAPE.  You will be presented with a series of three

menus  that simply  request  choices for  map projections,

their resolution and central focus region.

6.1  Word Map Projections Options Menu

You are given  the option of  three world map projections.

Actually,  there  are  five,  but  three  projections  are

combined in one display.  These options are illustrated in

Menu 8 below.

����������������������������������������������Ŀ

�            World Map Projections             �

�                                              �

�    1.  Mercator with Stereographic           �

�    2.  Hammer-Aitoff Equal Area              �

�    3.  Mollweide Equal Area                  �

�                                              �

�    Type option number and press Enter: _     �

������������������������������������������������

Menu 8.  World Map Projections.

6.1.1  Mercator with Stereographic Projections.

A  Mercator  projection  is  displayed  with stereographic

projections of both the  northern and southern hemisphere.

These projections are  perhaps the most  useful of all map

projections for navigation.   They have borne  the test of

time.

They  are  conformal projections,  that  is  they preserve

angles, so  that relative  shapes remain  the same.  Rhumb

lines are  straight on  the Mercator  projection.  Distor-

tions are  maximal in  the polar  regions of  the Mercator

projection,  but  minimal with  the  stereographic projec-

tions.   The   stereographic  projections   also  preserve

circles.  The combination compliments one another.

Trajectory Maker

User's Guide                                            38

����������������������������������������������������������

Examples of these projections are illustrated in Figure 9,

Section 7.

6.1.2  Hammer-Aitoff Equal Area Projection.

This  projection  preserves  areas and  is  best  near the

center  of  the  map.   It  is  similar  to  the Mollweide

projection described next,  but it has  less distortion in

the  polar  regions.   Far  east   and  west  are  not  as

distorted.  This  projection  is  useful  for  atlases  on

physical   geography  or   for   statistical  distribution

purposes.  It is used in astronomical work for showing the

distribution of stars.   An example of  this projection is

illustrated in Figure 10, Section 7.

6.1.3  Mollweide equal area projection.

This projection also preserves areas.  It is also known as

Babinet's  equal  surface   projection.   The  distortions

increase as you move away from the center, horizontally or

vertically.  Its major use is for geographic illustrations

relating to area such as density and populations or extent

of forests, etc.   It serves somewhat  the same purpose as

the Hammer-Aitoff projection.  An  example of this projec-

tion is illustrated in Figure 11, Section 7.

6.2  Map Resolution Options Menu

You are given three  map resolution options illustrated in

Menu 9 below.  Your trade-off is between speed and detail.

����������������������������������������������Ŀ

�            Map Resolution Options            �

�                                              �

�    1.  Fine ..... Spiffy but slow.           �

�    2.  Medium ... Good and faster.           �

�    3.  Course ... Fair and fastest.          �

�                                              �

�    Type option number and press Enter: _     �

������������������������������������������������

Menu 9.  Map Resolution Options.

Trajectory Maker

User's Guide                                            39

����������������������������������������������������������

6.2.1  Fine Option.

This option  gives the best  detail but is  the slowest of

the  three.   The data  file  contains  approximately 4500

latitude longitude pairs.

6.2.2  Medium Option.

This option gives  a good presentation  and is about twice

as  fast  as  the fine  option.   The  data  file contains

approximately 2000 latitude/longitude  pairs.  This option

is used in Trajectory Maker.

6.2.3  Coarse Option.

This option gives  fair presentation and  is the fastest -

about twice as  fast as the medium  option.  The data file

contains approximately 1000 latitude/longitude pairs.

6.3  World Map Focus Options Menu

The  final menu,  illustrated in  Menu  10, allows  you to

focus on one of three  regions of the world, provided that

you selected one of the equal area projections.

����������������������������������������������Ŀ

�               Word Map Focus                 �

�                                              �

�    1.  Greenwich Meridian                    �

�    2.  American Region                       �

�    3.  Soviet Region                         �

�                                              �

�    Type option number and press Enter: _     �

������������������������������������������������

Menu 10.  World Map Focus.

6.3.1  Greenwich Meridian.

The  central  meridian  is  the  Greenwich  meridian.  The

center of the map is at 0 degrees longitude.

Trajectory Maker

User's Guide                                            40

����������������������������������������������������������

6.3.2  American Region.

The central meridian is at 80 degrees west longitude which

puts the focus of the map between North and South America.

6.3.3  Soviet Region.

The central meridian is at 80 degrees east longitude which

puts the focus of the map in central Russia.

Trajectory Maker

User's Guide                                            41

����������������������������������������������������������

7.  TRAJECTORY SCAPE OUTPUT DESCRIPTION

If either the  American or Soviet  region is selected, the

displayed map will have a  yellow and a red meridian.  The

yellow meridian represents the  Greenwich meridian and the

red  meridian  represents  the  international  date  line.

These  two  meridians  provide   a  reference  base.   The

graticule is 20 by 20 degrees and is not labeled to avoid

clutter.

Once the map has been generated, the command window at the

bottom of  the display asks  for the name  of a geographic

trajectory file to display.  Simply  type the name of your

file,  and   the  program   will  display   the  orbit  or

trajectory.   If  you  store your  data  files  in another

directory,  give  the complete  path  specification.  Once

plotted,  you will  be  asked if  you  would like  to view

another trajectory.   The program will  continue until you

answer no.

To  aid  in   discrimination,  multiple  trajectories  are

displayed in different colors, a maximum of five.  You may

display  more trajectories,  however,  you are  limited to

five colors.

Examples  of the  various  projections are  illustrated in

Figures 9, 10 and 11.   Figure 9 illustrates four "tundra"

orbits  (Section   5.4)  on   the  conformal  projections.

Figures 10 and 11 illustrate the circular, 45 degree orbit

that  you generated  in  Section 2,  on  Hammer-Aitoff and

Mollweide projections, respectively.

Trajectory Maker

User's Guide                                            42

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 9                       �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            43

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 10                      �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            44

����������������������������������������������������������

����������������������������������������������������Ŀ

�                                                    �

�                     Figure 11                      �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

�                                                    �

������������������������������������������������������

Trajectory Maker

User's Guide                                            45

����������������������������������������������������������

8.  ADDITIONAL VIEWING CAPABILITY

In addition  to trajectory ephemerides,  you may construct

and view  circles, segmental arcs  and spherical polygons.

Because of  their area preserving  property, the Mollweide

and  Hammer-Aitoff  projections come  in  handy  for these

kinds of figures.  Also, recall that stereographic projec-

tions preserve circles.

8.1 Small Circles

Any plane intersecting  the Earth but  not passing through

its center will cut its  surface in a small circle.  Small

circles are useful for marking regions of interest such as

communication   regions  or   satellite   sensor  coverage

regions.

To construct  and view  small circles  on the  various map

projections,  when  prompted  for  the  file  name  in the

command window, press the F7 key.

Data fields  are presented for  the geographic coordinates

of the circle's center and its radius.  Enter the circle's

latitude (Lat),  longitude (Lon)  and its  radius (Rad) in

degrees.  Once the circle is  displayed, you will be asked

if you wish to  view another file.  If  you do, type y for

yes and repeat the above process.

8.2 Segmental Arcs and Spherical polygons

Any plane passing through the center of the Earth cuts its

surface in a great  circle.  A great circle  arc or arc is

defined by two points on  a great circle.  A segmental arc

is constructed from finite number  arcs joined in order in

such a  manner that  the initial  point of  each coincides

with  the terminal  point of  the  preceding arc.   If the

initial and terminal  points of a  segmental arc coincide,

it is  said to  be a  spherical polygon  (e.g. a spherical

triangle).

Segmental arcs  and polygons  are useful  for illustrating

minimal  distances  or  for  bounding  certain  geographic

regions  such   as  a  missile   threat  complex,  missile

corridors and abort regions.

To construct  and view  arcs, segmental  arcs or spherical

Trajectory Maker

User's Guide                                            46

����������������������������������������������������������

polygons (arc segments  of parallels of  latitude may also

drawn), when  prompted for  the file  name in  the command

window, press the F8 key.

Data fields  are presented for  the geographic coordinates

of  the  first  point.   Input  the  point's  latitude and

longitude in degrees.

Data fields are then presented  for the next point.  After

you have  entered the next  point, the  arc connecting the

two points is  drawn.  You may  continue constructing arcs

in  this  manner,  i.e.  by  entering  the  coordinates of

successive points,  to form  a segmental  arc or spherical

polygon.  When your  figure is complete,  press the F8 key

or the Esc key.

You will be  asked if you  wish to view  another file.  At

this time you may again press  the F7 key for a circle, or

the F8 key for a segmental  arc or simply type the name of

an  orbit file,  e.g. HOST.DAT,  that you  have previously

constructed with Trajectory Maker.

Trajectory Maker                          Historical Notes

User's Guide                                           A-1

����������������������������������������������������������

APPENDIX A.  HISTORICAL NOTES

In early  1976 while  employed under  the auspices  of the

Western Test  Range at  Vandenberg Air  Force Base, Calif-

ornia, the author was assigned the task of researching the

existing trajectory  related computer  programs in  use on

the  range,  and  assess the  relative  accuracy  of their

predicted impact  points based  on the  integrity of their

numerical integration  schemes.  Initially,  the study was

not to be  concerned with the force  model, but limited to

the integration process itself.

At that time, the author found that there existed at least

eleven  distinct  programs  lying  around  for propagating

reentry  trajectories.   Namely,  VPAR/BIPS, MIPS/AMARKIN,

FFTRG2, RIP, F&G, ARMS, OASYS,  FIT500, TGP, NITE and TAPP

(FIAT).  These  programs were  written in  FORTRAN for the

following computers: IBM 7094,  IBM 360/65, SIGMA 5/7, CDC

6400/6600, UNIVAC 1108, GE 635.

Their numerical integration schemes covered a fair gamut:

trapezoidal,  Taylor  series,   fixed  and  variable  step

fourth-order  Runge-Kutta  and  Runge-Kutta-Gill,  fourth-

order Adams-Moulton, eighth-order Gauss-Jackson, variation

of  parameters  and f  &  g series.   Moreover,  the force

models of the various programs varied.

For example, all of the models included at least two zonal

harmonic  coefficients,  in either  Jeffrey's  or Vinti's,

potential function.   One potential  function included six

zonal harmonics, and another included nine zonal harmonics

and  thirty-six tesseral  harmonics.  All  models included

atmospheric  drag  but  one used  the  U.S.  1962 Standard

Atmosphere,  while  another used  ARDC  1959.   There also

existed nonuniformity in the constant parameters.

All of the  models have their  merits and limitations, but

when  it comes  to  computational precision,  each  of the

things  above  must  be considered.   A  tenth  of  a foot

difference  in  the Earth's  equatorial  radius  may cause

unknown output  differences.  For  example, when comparing

the output of  two models, one  may observe differences in

the ninth  or tenth decimal  place, which could  be due to

computational  precision,  or  some  parameter difference,

among  other  things.  One  must  be able  to  isolate the

source of any discrepancy.

But the  primary thrust of  the study was  to evaluate the

Trajectory Maker                          Historical Notes

User's Guide                                           A-2

����������������������������������������������������������

numerical  integration  process   as  applied  to  reentry

trajectories.  Consequently, only  a  representative force

model was required.  One that would exhibit the underlying

behavior, when  the differential equations  of motion were

integrated.  This  model  should at  least  contain  a few

gravity harmonics, and  more importantly, a representative

atmospheric  drag model,  which is  what really  taxes the

integrator.

The  problem then,  became one  of isolation.   A baseline

integrator was  required, whose  precision was  known with

certainty, in order  to compare the  integrators.  Also, a

representative  force  model   was  required  that  could,

without  too  much  difficulty,  be  substituted  into the

various trajectory programs, including the baseline model.

The force  model didn't need  to be all  that exotic, just

representative  of the  kinds  of forces  the differential

equations would experience.

The gravity  model chosen for  the baseline  model was the

one  developed  by  R.M.L.   Baker  at  Computer  Sciences

Corporation for use  in their f &  g series approach.  The

reason for this was that their program was one of interest

to some range people, and their  f & g series approach was

based on high  order derivatives of  the gravity potential

function, which were  embedded in the  program code.  This

model  included   the  zonal  gravity   harmonics  of  the

Vinti-potential of order six.

The  drag force  model selected  was  based on  the abbre-

viated,  preliminary tables  for the  U. S.  1976 Standard

Earth Atmosphere.   The reason for  this was  that most of

the atmospheres  used in  the various  models were lengthy

tables.  The abbreviated tables contained just 21 pairs of

altitude versus  density, which were  more manageable, and

could be  accurately modeled with  cubic spline functions.

In addition, the model was the latest and greatest.

The numerical  integration scheme chosen  for the baseline

integrator was the  eighth-order, fixed-step, Runge-Kutta-

-Shanks' technique  having twelve stages.   This scheme is

stable, and was  the most accurate  scheme that the author

knew of at the time.

A  fixed-step  scheme  was  desirable,  because  one could

determine  precisely  the  accuracy  of  the computations.

This could be  done by selecting  a step-size, integrating

the differential  equations to a  predefined time, cutting

Trajectory Maker                          Historical Notes

User's Guide                                           A-3

����������������������������������������������������������

the  step-size  in  half,  repeating  the  integration and

comparing  the two  trajectories.   This process  could be

continued  until  two successive  trajectories  matched in

their fifteenth or sixteenth significant figures (the pre-

cision of double precision  arithmetic on the IBM 360/65).

Clearly, computation was of no concern - just accuracy.

Once  the  baseline  force  model  together  with  all the

relevant   constants  was   substituted  into   the  other

trajectory  programs,  the accuracy  of  their integrators

could  be  determined  with a  fair  amount  of certainty.

Certainty could be claimed if  it wasn't for the fact that

the  various hardware  on which  the software  resided had

some differences in precision.

It  was  soon  discovered  that  the  baseline  model  was

producing  rather  accurate  trajectories.   In  fact,  by

shifting  density  models  it  was  proven  that  the USAF

"certified" program,  FFTRG2, could miss  impact points by

as much as two nautical  miles, just because of the manner

in which it was interpolating the density profile.*  More-

over, its interpolation scheme, used presumably for compu-

tation  speed,  actually  slowed  the  integrator  down by

causing  the  step-control  to  artificially  decrease the

step-size.

Considerations such as  these led to  many enhancements of

the  force  model  in  the  baseline  program.   The final

version included the following:

o  Six   zonal   gravity   harmonic   coefficients.

(Smithsonian 1973 II values)

o  U. S. 1976 Standard Atmosphere.

Fitted with cubic splines to the logarithm of

the density profile.

o  Drag  coefficient   as  a   function  of  either

altitude,  or  Mach number.   Fitted  with cubic

splines or linear depending on the profile.

���������������������

*Ivy,  L. H.  and H.  B. Reynolds,  On the  Sensitivity of

Impact Prediction to  Methods of Interpolating Atmospheric

Density, ITT Federal  Electric Corporation, Technical Note

TN-76-1463, Vandenberg Air Force Base, CA, Dec 22, 1976.

Trajectory Maker                          Historical Notes

User's Guide                                           A-4

����������������������������������������������������������

o  Accurate speed of sound computation based on the

1976 temperature profile.

o  Atmospheric  wind profiles.   Fitted  with cubic

splines.

o  Innovative  altitude computation  which produces

trigonometric  functions for  wind  effects, and

the topocentric rotation matrix as a by product.

The  baseline  integrator,  i.e.  the  Runge-Kutta-Shanks'

fixed-step scheme, proved  to be very  accurate.  To speed

things up, a dual mesh step control option was added.

What started  out to  be an  accuracy study  for numerical

integration   schemes  of   various  Western   Test  Range

trajectory  programs  ended  up  being  another trajectory

program  -  a  rapid,  high  precision  reentry trajectory

generator.  And that program is the basis of Trajectory

Maker.

Later, the  author joined  another aerospace  company, and

shortly  thereafter, an  assignment required  a trajectory

program.  The company's  program (actually, their program,

RIPR, a variation of  RIP, was bootlegged from Vandenberg)

would  go  unstable  for  certain  trajectories  when they

entered the atmosphere.   This led to  a conversion of the

baseline  program to  run on  their in-house  PDP-11.  The

baseline  program  was  then  used  for  validating  Space

Shuttle ephemerides, and  with its use  an error in launch

azimuth was uncovered in a NASA simulation program.

Another   aerospace  company   had   spacecraft  computers

in-house for use in  verifying, validating, and testing of

the  flight  software  for  a  particular  USAF  satellite

system.   In addition  they  had a  separate  computer for

simulating the spacecraft's environment.

Several problems were manifested  that required the use of

an  independent   trajectory  simulation.    The  baseline

program  was  again  modified for  use  on  their in-house

Perkin Elmer  8/32 computer.  This  time, all  of the drag

stuff was  stripped out of  the program,  and emphasis was

placed on the integrator and gravity model.  Clever closed

form expressions  were developed  and implemented  for the

gravity model so that it would include an arbitrary number

of harmonics with rapid computation times.

Trajectory Maker                          Historical Notes

User's Guide                                           A-5

����������������������������������������������������������

The program  was used  for several  applications including

another  satellite  program.   The  baseline  program  was

further vindicated  when it matched  orbits generated with

the  Advanced  Orbit  Ephemeris Subsystem  in  use  at the

Satellite Control Facility at Sunnyvale, California.

The program was also used for Strategic Defense Initiative

applications.   In particular,  it  was used  to propagate

thousands of missile threat trajectories into the field of

view of  special sensors.  It  was also used  to study the

following:

o  A Space based kinetic energy weapon interceptor

system.

o  Evasive enemy spacecraft  orbit maneuvers in the

Earth's shadow.

o  Multi-satellite   constellation  design   for  a

special sensor.

o  Drag  effects on  a  space based  rotating radar

antenna system.

o  Relative  motion  for  a  space  laser targeting

experiment.

Having reviewed the merits and limitations of the baseline

program relative to the many  applications in which it has

been  involved, and  modifications  it has  undergone, the

author decided to redo the entire program from a different

perspective, and in a different language.

There exists a  number of software  tools available in the

market today that are  neat programs for orbital mechanics

and astrodynamics applications, and several were purchased

at one  of the  recent companies  where the  author was an

employee.  But the fact of the matter is, all of them were

looked at briefly,  and put on  a shelf, for  none of them

did what was needed to be done.

Invariably, it was the case, that a special tool needed to

be developed to  solve the problem  at hand.  Moreover, it

was usually  the case that  individual trajectories needed

to be generated  and plotted (usually  plotted by hand) in

order  to shape  the underlying  problem. The  most useful

tool has been the capability  to generate a trajectory, of

varying degrees  of complexity, depending  in the applica-

Trajectory Maker                          Historical Notes

User's Guide                                           A-6

����������������������������������������������������������

tion, and see what it looked like.

Trajectory  Maker is  the result  of selecting  and adding

those features  that the author  has found to  be the most

useful in many aerospace applications requiring the use of

a trajectory simulation.

Trajectory Maker                                  Glossary

User's Guide                                           B-1

����������������������������������������������������������

APPENDIX B.  GLOSSARY

This appendix contains  definitions of terms  used in this

User's Guide.  Although many of these terms are applicable

for  other celestial  bodies,  the definitions  herein are

given in reference  to planet Earth.   For example, we use

perigee rather than periapsis.

Altitude  The height of an object above the surface of the

Earth.

Apogee  The point on an elliptical orbit farthest from the

Earth's center.

Argument of perigee  The  angular distance measured in the

orbit plane  in the direction  of motion from  the line of

nodes to the line of apsides.

Ascending  node   The  node  within  the  equatorial plane

through which an object passes from South to North.

Azimuth  The angular  measure in the  plane of the horizon

from  the  north point  on  the horizon  clockwise  to the

object.

Ballistic coefficient   The mass  of an  object divided by

the  product of  its drag  coefficient with  its effective

cross sectional area.

Ballistic trajectory  A trajectory that is in free-flight,

having no power applied to it, that intersects the surface

of the Earth.

Celestial sphere   An imaginary  sphere with  the Earth at

its center, on which all objects in the sky seem to lie.

Conformal  projection  A  projection  of a  sphere  onto a

plane that preserves angles.

Conic section   One of three  curves formed  by the inter-

section of a plane with a cone.  They are: (1) an ellipse;

(2) a parabola; (3) an hyperbola.

Critical inclination   The inclination  of an  orbit where

the  perigee point  neither  advances or  regresses.  This

occurs at 63.44 degrees.

Trajectory Maker                                  Glossary

User's Guide                                           B-2

����������������������������������������������������������

Declination  The  angular distance  of an  object North or

South of the equator.

Descending  node   The node  within  the  equatorial plane

through which an object passes from North to South.

Differential equation  A mathematical equation involving

derivatives, e.g. the equations of motion.

Drag  The  force acting  on an  object due  to the Earth's

atmosphere.  Drag acts in a  direction opposite to that of

the objects motion relative to the atmosphere.

Drag  coefficient   A  dimensionless  coefficient  used in

characterizing the atmospheric drag  force.  A good number

is 2.2.

Eccentricity   A  parameter  that  characterizes  a  conic

section: ellipse, parabola, hyperbola.

Ecliptic  The path  that the Sun  appears to follow around

the celestial sphere each year.  The ecliptic approximates

a great circle  inclined about 23.5  degrees to the celes-

tial equator.

Elevation   The  angle an  object  is above  or  below the

horizon.

Ellipsoid   A  surface  of   revolution  generated  by  an

ellipse.

Ephemeris  A table of  predicted positions for a celestial

body.  As  used herein,  the table  may include velocities

and accelerations as well.

Escape velocity   The speed at  which an  object will just

break free from the gravitational  pull of the Earth.  The

value  is about  11  km/sec.  The  path  is a  parabola or

hyperbola depending on the precise value.

Flight path angle  The angle between the inertial velocity

vector an the local horizontal plane.

Geocentric latitude  The angle at  the center of the Earth

between the radius through a given place or object and the

equatorial plane.

Trajectory Maker                                  Glossary

User's Guide                                           B-3

����������������������������������������������������������

Geodetic  latitude   The  angle between  a  normal  to the

reference ellipsoid at the point and the equatorial plane.

Graticule  The network of  lines of latitude and longitude

upon which a map is drawn.

Gravitational Harmonics   Terms appearing  in the gravita-

tional potential function (zonal, sectorial and tesseral)

Great circle  Any circle on  the celestial sphere that has

the Earth at its center.

Greenwich  Mean  Time  (GMT)    Mean  solar  time  on  the

Greenwich meridian.  Also known  as Universal Time (UT) or

Zulu time (Z).  It may  be determined from local zone time

by  adding   (when  West)   or  subtracting   (when  East)

longitude.

Greenwich meridian  The zero meridian from which geograph-

ical longitude is measured,  passing through the Greenwich

observatory, England ( also called the prime meridian).

Ground trace  The path on  the surface of the Earth traced

by an orbiting object's  subvehicle point, i.e., the point

directly below the object.  That  is, the path along which

an object passes  directly overhead as  seen from the sur-

face of the Earth.

Hammer-Aitoff projection  A  map projection that preserves

area.  Its graticule is  constructed from curved parallels

and meridians  which reduce  distortions near  the edge of

the  map.   First  described  by  Hammer,  a  professor of

surveying  at  Stuttgart,  in 1892  following  an  idea of

Aitoff's.

Hour angle  The angle  between the observer's meridian and

the hour circle that passes through the object.

Hour circle  A great circle  that passes through an object

on the celestial sphere and the celestial pole.

Inclination   The angle  between the  orbit plane  and the

equatorial plane.

Injection  The  act of placing  an object  on a calculated

trajectory.

Trajectory Maker                                  Glossary

User's Guide                                           B-4

����������������������������������������������������������

Insertion  The act of placing an object into orbit around

the Earth.

Kepler flight  time  The  time of  flight for  a ballistic

missile, describing  an elliptical  orbit, to  impact with

the Earth's surface.

Keplerian motion  Motion that obeys Kepler three laws: (1)

The orbit of each planet is an ellipse with the Sun at one

focus.  (2) The radius vector  joining the Sun to a planet

sweeps over equal  areas in equal  intervals of time.  (3)

The ratio of the squares of the periods of any two planets

is equal to the ratio of the cubes of their mean distances

from the Sun.

Latitude   The  angular  distance north  or  south  of the

equator.  See geodetic latitude above.

Line of apsides  A line  connecting the perigee and apogee

points.

Line of  nodes  The  intersection of  the equatorial plane

and the orbit plane.

Local horizontal   The plane  normal to  the radius vector

that contains the object.

Longitude  The  angle between  the Greenwich  meridian and

the meridian through  a place or  object, measured East or

West.   Geocentric  longitude  is  the  same  as  geodetic

longitude.

Longitude of the ascending node  The angular distance from

the vernal  equinox measured  eastwards in  the equatorial

plane  to the  point of  intersection  of the  orbit plane

where the object crosses from South to North.

Mercator projection  A conformal map projection introduced

by Gerhard Mercator in 1569.  Loxodromes ( rhumb lines) on

the sphere are mapped into straight lines on the map.

Mercator's projection is the only one that does this.

Meridian   A great  circle passing  through the  North and

South poles.

Mollweide  projection   A  map  projection  that preserves

area.  The  graticule is  constructed from  parallels that

are  unequally   spaced  straight   lines  and  elliptical

Trajectory Maker                                  Glossary

User's Guide                                           B-5

����������������������������������������������������������

meridians.  The central  meridian is a  special case where

the ellipse degenerates  into a straight  line.  The 90 th

meridian  is  the  other special  case  where  the ellipse

becomes a  circle.  First  described by  Karl Mollweide, a

Mathematician and Astronomer, in 1805.

Nodes  The points of intersection of an orbit with and the

equatorial plane.

Object  A body, satellite, spacecraft, missile, etc.

Orbit  The path of one body in space around another.

Orbital element  Six quantities that describe the orbit of

a body  in space.  They  are: (1) the  semimajor axis; (2)

eccentricity;  (3)  inclination;   (4)  longitude  of  the

ascending node; (5)  argument of perigee;  (6) the time of

perigee passage.

Oblate  spheroid   A  surface  generated  by  rotating  an

ellipse about its semiminor axis.

Perigee  The point in an orbit nearest the Earth's center.

Period  The time  required for one  complete circuit of an

orbit.

Perturbation  Deviation from  exact reference motion (e.g.

conic  section)  caused  by  gravity  anomalies  or  other

forces.

Potential  function   At  a point,  the  work  required to

remove a unit mass from that point to infinity.

Radius vector  The vector emanating from the center of the

Earth to the orbiting object.

Range  The distance from a radar site to an object.

Range rate  The time rate of change of radar range.

Right ascension   Angular distance  of an  object measured

eastwards  along  the celestial  equator  from  the vernal

equinox  to the  great  circle passing  through  the north

celestial pole and the object.

Trajectory Maker                                  Glossary

User's Guide                                           B-6

����������������������������������������������������������

Sidereal time  The hour angle of the vernal equinox.

Spheroid   An oblate  spheroid which  closely approximates

the mean sea-level figure of the Earth's geoid.

Stereographic projection   A circle  preserving, conformal

map projection  obtained by  projecting the  points of the

sphere from the  North pole onto the  tangent plane at the

South pole (or vice  versa).  Meridians pass into straight

lines, parallels  into concentric  circles.  It  may be to

Hipparch (c. 150 B.C.).

Sun synchronous  An  orbit in which  the angle between the

orbit  plane and  the line  joining the  Sun and  Earth is

constant.  An object in such  an orbit enjoys passing over

the same part of the Earth at the same time each day.

Trajectory  The  path of  a body  in space  or through the

Earth's atmosphere.

Universal  time    Mean  solar  time   referenced  to  the

Greenwich meridian.

Universal variable  A variable that bridges the transition

from  closed  to open  orbits  in the  two-body  theory of

motion.   Universal  because  one  variable  replaces  the

awkwardness of using three  distinct formulations for each

kind of conic section.

Velocity azimuth angle  The  angle measured clockwise from

the  North  to  the projection  of  the  inertial velocity

vector onto the local horizontal plane.

Vernal equinox  The point  of intersection of the ecliptic

and celestial  equator where  the Sun  crosses the equator

from South  to North in  its apparent  annual motion along

the ecliptic.  This event occurs near March 21.

Zulu time  Greenwich Mean Time or Universal Time.