To install MITE on the Sanyo 1000 with SMARTMODEM

1. Set switches inside SMARTMODEM front panel as follows:

1 - down (unless no parameter changes are expected)

2 - up

3 - down

4 - up

5 - up (if auto answer desired)

6 - up

7 - up (if normal residential phone line)

8 - doesn't matter

2. Build RS-232 cable as follows:

2 ----- 2

3 ----- 3

4 ----- 4

5 ----- 5

6 ----- 8	NOTE: lines 6 and 8 are flipped

7 ----- 7

8 ----- 6

20 ----- 20

3. Connect either end to SMARTMODEM, other end to Sanyo

4. Set baud rate to 1200, as described in Sanyo manual.

MITE will select between 300 and 1200 under software

control. Help from a technician may be required to

perform this step.

5. Copy MITE/S.COM to your own diskette, to avoid accidental

loss of the master copy.

6. Read through entire MITE manual. If possible have a copy

of MITE running on a system in front of you.

7. Your macro string index characters have been installed as

the characters 0 through 9, respectively.

8. Note that the DTR auto-hangup feature causes problems with

dynamically changing parameters, as the 8251 drops DTR every

time you go to change certain of the parameters. If you do

not plan to change things once you are logged in (and do not

plan to transfer binary data files, which automatically does

parameters changes), then switch 1 can be left UP, which will

allow DTR controlled hangup to work. In any case, you can

always make the Smartmodem hangup by typing three plusses

followed by the command AT H0 (see Smartmodem manual). NOTE:

Switch 1 must be UP for BYE.COM to function normally (which

includes binary file transfers with XMODEM).

##########################################################################################################