.PAGE LENGTH 66

.ABOVE HEADER 1

.BELOW HEADER 3

.ABOVE FOOTER 2

.BELOW FOOTER 3

.PAGE BREAK

.PAGE NUMBER 1

.HEAD/Page  1.#          WatchWord Printer Configuration            Page  1.#/

.FOOT//

.NOMJ

WatchWord can be used to support a variety of printer features including

underlining, boldface, strikeout, superscripts, subscripts, emphasized,

compressed, alternate character fonts, and microjustification.  The method in

which each of these features is implemented depends on the type of printer you

have.

Three methods for underlining are available, all turned on or off with CTRL-W.

Hardware underlining refers to the use of a control sequence sent to the

printer to turn on or off underlining.  Many dot matrix printers support this

feature.  The underlining is part of the hardware (or firmware) of the printer.

Backspace underlining refers to issuing a backspace followed by an underbar

character (_) after each character to be underlined.  Reprint underlining

refers to issuing a carriage return without a line feed at the end of a line

requiring underlining and printing underbar characters in the appropriate

places.

Similarly, there are many possible boldface implementations.  Some printers

support several types of hardware boldface.  This is usually referred to as

emphasized printing, and for most of the supported printers WatchWord uses

CTRL-Q for turning this on or off.  These printers usually also allow boldface

to be done with software by overprinting characters.  Overprinting can be done

either with a backspace or by reprinting the appropriate portion of the line

using a carriage return without line feed.  Those printers for which

microjustification is supported also allow boldface to be done with a slight

offset causing a broadened character.  The commands BOLDFACE OFFSET and BOLDFACE

OVERPRINT are used to choose between these.  When overprint is used, the number

of times overprinting occurs can be set with the BOLDFACE n command.

Superscripts and subscripts are handled either by moving the carriage up and

down at the appropriate place (software) or by using a hardware feature of the

printer.  The hardware feature is chosen by default for those printers which

support it.  Software superscripts and subscripts require that the paper can be

moved down as well as up.  For this to look good, the carriage should be moved

by half a line.

Microjustification is currently only supported for Diablo and Spinwriter type

printers and requires that the printer be able to advance 1/120 or 1/60 of an

inch.

WatchWord normally calls the ZDOS BIOS directly to handle printer output.  This

is because of a flaw in MS-DOS which does not allow the user to query the

printer to see if it is ready.  Once MS-DOS sends a character to the printer, it

will not return until it has been sent.  If your printer runs out of paper, your

program would hang up until the condition is fixed.  Even worse, for some

configurations, the program could hang up if no printer is attached.  Calling

the BIOS directly solves this problem.  WatchWord will inform you if the printer

is not ready (after a short delay).  You may abort at this time (with CTRL-C) or

do nothing and wait for the printer to become ready.  The use of the BIOS for

printer output may interfere with certain specialized drivers for some printers.

The numbered command, 51233, will turn off the BIOS calling for printer output

and use ZDOS function calls instead.  The SHOWP command will show whether the

BIOS is being called for printer output so you can see if you executed the

command correctly.

.START PAGE

Many printers are in use and it would not be possible for WatchWord to directly

support all of them.  The printers that are directly supported by WatchWord are

described below.  To use one of these printers, just type the command which is

given below, or use the WWCONFIG.EXE program to permanently configure WatchWord

for the printer of your choice.  If your printer is not one of the types listed

below, you can still use the WWCONFIG program to configure WatchWord to your

printer.

DUMB

This command is used with print wheel type printers and dot matrix

printers which support no features.  It allows underlining, boldface, and

strikeout using reprinting.

SPINWRITER

This is for the Spinwriter and Spinwriter compatible printers.  Supported

features are superscripts, subscripts, boldface with overprint or offset,

underlining, strikeout, 6 or 8 lines per inch, 10 or 12 pitch, and

microjustification using 1/120 inch spacing at 10 or 12 pitch.

DIABLO

This is for the Diablo and Diablo compatible printers.  Note that some

models of the NEC Spinwriter such as the 5525 are designed to be Diablo

compatible, and this command should be used for these printers.  Supported

features are superscripts, subscripts, boldface with overprint or offset,

underlining, strikeout, 6 or 8 lines per inch, 10 or 12 pitch, and

microjustification using 1/60 inch spacing at 10 or 12 pitch.

EPSON

There are several models of the EPSON printer and many EPSON compatible

ones.  The different models have different features, and so not all of the

features described will be available on all models.  Supported are hardware

superscripts and subscripts, hardware underlining, 6 or 8 lines per inch,

10 or 12 pitch, compressed printing using CTRL-O, expanded printing using

CTRL-P, emphasized printing using CTRL-Q, double strike using CTRL-R,

italics using CTRL-S, and down loaded font using CTRL-T.  Underlining and

strikeout using reprinting is also supported.

PROWRITER

Supported are hardware superscript and subscript, hardware underlining, 6

and 8 lines per inch, 10 and 12 pitch, compressed printing using CTRL-O,

elongated printing using CTRL-P, boldface using CTRL-Q, proportional

printing using CTRL-R, Greek using CTRL-S, and graphics characters using

CTRL-T.  Not all models of the printer support all of these features.

.NEED 5

MPI

Supported are boldface and strikeout using reprinting, underlining using

reprinting or hardware underlining (the default), 6 or 8 lines per inch,

and elongated printing using CTRL-P.

OKIDATA (Okidata 82 or 83)

Supported are underlining, boldface, and strikeout using reprinting, 6 or 8

lines per inch, compressed using CTRL-O, and expanded using CTRL-P.

OKIDATA9 (Okidata 92 or 93)

Supported are hardware superscripts and subscripts, backspace, boldface,

strikeout, underlining using reprinting, hardware underlining (the

default), 6 or 8 lines per inch, 10 or 12 pitch, compressed printing using

CTRL-O, expanded printing using CTRL-P (returns to 10 pitch), emphasized

printing using CTRL-Q, enhanced printing using CTRL-R and correspondence

quality (returns to data processing mode).

GEMINI

Supported are hardware superscripts and subscripts, backspace, underlining

using reprinting or hardware underlining (the default), 6 or 8 lines per

inch, 10 or 12 pitch, compressed printing using CTRL-O, double width

printing using CTRL-P, emphasized printing using CTRL-Q, double strike

using CTRL-R, italics using CTRL-S, down loaded characters using CTRL-T.

MANNESMANN

This is used for the Mannesmann Tally printers.  ANSI sequences are used so

that this command may be used for other printers using ANSI standard

control sequences.  Supported are hardware superscripts and subscripts,

underlining using reprinting, hardware underlining (the default), 6 and 8

lines per inch, 10 and 12 pitch, and emphasized printing using CTRL-Q.

H25

This is for the H25 and Z25 printer.  Supported are superscripts and

subscripts using paper movement of a full line, underlining, boldface and

strikeout using reprinting, 6 or 8 lines per inch, 10 or 12 pitch, and

graphics characters using CTRL-T.

.START PAGE

WatchWord is designed so that most other printers can be used comfortably,

after some user configuration.  Of course, if all you wish to do is straight

output of text, no configuration is necessary other than configuration under

ZDOS.

WatchWord keeps track of two sets of control sequences.  The sequences in the

first set are called the STANDARD PRINT SEQUENCES and each has a predefined

meaning as described below.  The sequences in the second set are referred to as

USER PRINT SEQUENCES and can be used in any way you see fit.  (See below for the

procedure to set these sequences.)

The sequences in the first set are:

Number Name                    Description

0	start superscript   one half reverse line feed

1	end superscript     undo above (usually one half line feed)

2	start subscript     one half line feed

3	end subscript	    undo above (usually one half reverse line feed)

4	backspace	    backspace (usually a backspace character)

5	underline	    backspace and print underbar

6	strikeout	    backspace and print hyphen

7	6 lines per inch    set to 6 lines per inch

8	8 lines per inch    set to 8 lines per inch

9	10 pitch	    set to 10 pitch

10	12 pitch	    set to 12 pitch

11	microspace	    set so that carriage moves 1/120 inch per character

12	vertical pitch	    vertical pitch sequence

13	horizontal pitch    horizontal pitch sequence

14	return to default   return to printer switch settings

15	start underline     hardware start underline sequence

16	stop underline	    hardware stop underline sequence

17	CTRL-O start	    sequence started by CTRL-O

18	CTRL-O end	    sequence ended by CTRL-O

19	CTRL-P start	    sequence started by CTRL-P

20	CTRL-P end	    sequence ended by CTRL-P

21	CTRL-Q start	    sequence started by CTRL-Q

22	CTRL-Q end	    sequence ended by CTRL-Q

23	CTRL-R start	    sequence started by CTRL-R

24	CTRL-R end	    sequence ended by CTRL-R

25	CTRL-S start	    sequence started by CTRL-S

26	CTRL-S end	    sequence ended by CTRL-S

27	CTRL-T start	    sequence started by CTRL-T

28	CTRL-T end	    sequence ended by CTRL-T

Sequences 12 and 13 are not currently supported.

Sequence 4 is needed for overprinting.  If this sequence is empty, then

boldface, underlining, and strikeout will be done by reprinting, except that

underlining will be done using sequences 15 and 16 if hardware underlining

is set.

.NEED 3

Sequence 11 is used for microjustification as well as boldfacing done with

offset.  It is currently only supported for the DIABLO and SPINWRITER printers.

.NEED 3

Note that the PITCH10 or PITCH12 command not only outputs sequence 9 or

10, but also informs WatchWord of the pitch so it can calculate the

appropriate number of microspaces during microjustification.

.NEED 4

WatchWord allows several types of boldface.  Three types of software boldface

are available.  Boldface by overprinting can be done on any printer supporting

the backspace character.  Boldface by reprinting only requires the carriage

return to be supported.  Overprinting will be used if sequence 4 is not empty.

The BACKSPACE and NOBACKSPACE commands can be used to fill or empty sequences

4, 5, and 6.  You can set the number of extra strikes per character during

these types of boldface with the BOLDFACE command.

BOLDFACE n

will cause n extra strikes per character.  If you wish to use the third type of

boldface, the printer must be able to both backspace and print a microspace.

Sequence 11 must therefore be set for your printer.  Sequence 14 must be set in

order to use this type of boldface.  The command:

BOLDFACE OFFSET

tells WatchWord to use a small offset to produce boldface.  You can return to

one of the first two types of boldface with the command:

BOLDFACE OVERPRINT

CTRL-X is used to turn on and off software boldface.  In the ENHANCE modes,

software boldface is normally displayed using reverse video.  If the HWB

command (hardware boldface) is executed, STANDARD PRINT SEQUENCES 23 and 24

will be used to turn on and off boldface at the printer, rather than using one

of the three types of software boldface described above.  CTRL-X can still be

used to turn on or off boldface.  If software boldface is selected (command

number 51322), then CTRL-Q can be used to turn on or off the software boldface

(overprint, reprint, or offset).  The main difference between using CTRL-X and

CTRL-Q is that CTRL-X displays the boldface in reverse video in the ENHANCE

modes while CTRL-Q can be used will auxiliary fonts.  A boldface font is

included with WatchWord for this purpose.

The sequences described above can be set either using WWCONFIG.EXE or

interactively from WatchWord.  See the file WWCONFIG.DOC for use of

WWCONFIG.EXE.  From WatchWord the sequences are set using numbered commands.

The user should read the first part of the file NUMBER.DOC to become familiar

with the use of numbered commands.  The numbered commands for setting the

sequences described above have consecutive numbers starting with 52736.  (This

number can be found in the table of numbered commands.)  The syntax for setting

a print sequence is:

#n/string/

where n is the command number, and "string" is the character string you wish

the sequence to have.  Since this string will normally contain non-printing

characters you will probably have to enter it using the SPECINS key (SHIFT-F1).

For the standard sequences, the command number, n is determined by adding the

base number (52736) to the number given in the standard sequence list above.

For example, to set the control sequence for 6 lines per inch, the command

number would be 52743 (52736 + 7) and to set this to the two character

sequence, ESC 2, we would enter the command on the COMMAND line as:

#52743/<SHIFT-F1><ESC>2/

There are 30 additional control sequences which the user can set to suit

the individual needs of the printer.  These can also be set either from

inside of WatchWord or by using WWCONFIG.EXE.  For each of these sequences

there is a numbered command to set the sequence (with the syntax above) and

a numbered command (with no arguments) to output the sequence to the

printer.  The command numbers for setting these commands are 52992-53021,

and the numbers for outputting the corresponding sequence are 53248-53277.

Note that two command numbers corresponding to the same sequence differ by

256.

.NEED 7

Dot commands can be used to output any of the 30 control sequences described

above.  These have a restriction of being sent between lines of text.  WatchWord

provides a method of sending control sequences at any time, even in the middle

of a line.  Of course, one method would be to directly insert the desired

control sequence at the appropriate place in the text.  WatchWord provides a

printer map feature which avoids this necessity.

WatchWord allows you to map any character into any sequence of characters

when being output to the printer.  This sequence only affects printer

output.  A PMAP command is provided for this and is described in the manual.

.END PAGE