CompatiCard II BIOS Notes -- Version 2.02 -- March 31, 1992

The following information is provided for programmers that need

to access diskettes sectors directly.  This information is

provided with no technical support for use by qualified

programmers.  They are similar in function to the routines

provided by the system BIOS which you should be familiar with.

Any comments or suggestions should put in writing and sent Attn:

Software Engineering.  Reasonable requests for additional

functions may be incorporated into future versions.

CompatiCard II BIOS Interrupt 13h Specifications

These functions are provided by the CompatiCard II BIOS

(contained in CCDRIVER.SYS) when properly installed.  The

CompatiCard II functions are similar to the system BIOS functions

provided for floppy diskette I/O.  When an interrupt 13h function

is requested, the CompatiCard II BIOS will look at the physical

drive address specified and it will only process requests with

valid addresses.  An address is valid if it was specified in the

CCDRIVER.SYS line of the CONFIG.SYS file.  Any interrupt 13h

requests that do not contain valid addresses will be ignored and

passed to the system BIOS for processing.

Most of the functions return an error code in the AH register

upon completion.  In addition, the carry flag (CY) will be set if

the error code returned in AH is non-zero.  The error codes are

defined as follows:

AH = 00h  successful completion of operation

80h  diskette drive was not ready

40h  seek failure

20h  general controller failure

10h  data error (CRC) on diskette

09h  attempt to DMA across 64K boundary

08h  DMA overrun

04h  sector not found

03h  write protect error

02h  address mark not found

01h  bad command passed to driver

Some of the functions reference parameters that are kept in the

BIOS diskette drive parameter table.  The table is pointed to by

INT 1eh (0:78h).  The layout of the table is as follows:

Offset  Size   Description

0    byte   not used

1    byte   not used

2    byte   Number of timer ticks to keep motor on

3    byte   bytes per sector (0=128, 1=256, 2=512, &

3=1024)

4    byte   number of sectors on a track (used only by

the FORMAT function)

5    byte   read/write/verify gap length

6    byte   not used

7    byte   format gap length

8    byte   format fill byte

9    byte   head settle time (in milliseconds)

10    byte   motor startup time (in 1/8 seconds)

If your program needs to alter any of these parameters from their

original values, you should copy the original table to a scratch

area and re-point INT 1eh to the scratch area.  The alterations

should be made to your copy of the table and not the original.

Before your program terminates, you must restore INT 1eh back to

the original table.

Functions:

0 - reset disk system

This function can be called after a disk error.  It is

not required because the CompatiCard II BIOS will

automatically perform a reset when needed.

Entry:

AH = 0

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

1 - Get I/O error code from last operation

This function will return the status code from the last

I/O operation performed on the drive.

Entry:

AH = 1

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

2 - Read sectors

This function is used to read from a diskette.  It can

be used to read more than 1 sector but all of the

sectors to be read must be on the same track.

Entry:

AH = 2

AL = number of sectors to read (1-n)

ES:BX = address of buffer

CH = cylinder (0-n)

CL = sector number

DH = head (0-1)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

AL = number of sectors successfully read

Note: Sector size, motor on time, motor startup time,

head settle time, and gap length are obtained from the

BIOS diskette drive parameter table.

3 - Write sectors

This function is used to write to a diskette.  It can

be used to write more than 1 sector but all of the

sectors to be written must be on the same track.

Entry:

AH = 3

AL = number of sectors to write (1-n)

ES:BX = address of buffer

CH = cylinder (0-n)

CL = sector number

DH = head (0-1)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

AL = number of sectors successfully written

Note: Sector size, motor on time, motor startup time,

head settle time, and gap length are obtained from the

BIOS diskette drive parameter table.

4 - Verify sectors

This function is used to verify sectors from a

diskette.  It can be used to verify more than 1 sector

but all of the sectors to be verified must be on the

same track.

Entry:

AH = 4

AL = number of sectors to verify (1-n)

CH = cylinder (0-n)

CL = sector number

DH = head (0-1)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

AL = number of sectors successfully verified

Note: Sector size, motor on time, motor startup time,

head settle time, and gap length are obtained from the

BIOS diskette drive parameter table.

5 - Format track

This function is used to format a track on a diskette.

Entry:

AH = 5

ES:BX = ptr to buffer containing sector information

CH = cylinder (0-n)

DH = head (0-1)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

Note: Sector size, format gap length, number of sectors

to format, and fill byte are obtained from the BIOS

diskette drive parameter table.

The sector information required for formatting is a

collection of four byte address fields.  There should

be one address field per sector being formatted.  Each

field is made up of four bytes, (C, H, R, N), where

C=track number, H=head number, R=sector number, and

N=number of bytes per sector (0=128, 1=256, 2=512, and

3=1024).

8 - Return drive parameters

This function is used to return some parameters about

the specified drive.

Entry:

AH = 8

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

AX = 0

BL - bits 7 to 4 = 0

bits 3 to 0 - drive type

0 = other type

1 = 360KB

2 = 1.2MB

3 = 720KB

4 = 1.44MB

BH = 0

CL = maximum sectors per track

CH = maximum number of cylinders

DL = number of primary drives installed

DH = maximum head number

ES:DI = pointer to 11 byte diskette drive parameter

table associated with the maximum supported

media type for the drive specified

CY = 1 if error

AH = error code

9 - Set CompatiCard parameters

This function is used to set parameters for a diskette

drive.  Each physical drive get its own set of

parameters which are retained unless they are changed

using function 9 or DOS is re-booted.

Entry:

AH = 9

AL = number of retries after I/O error (0-255)

CH = double step flag (0=single step and  1=double

step)

CL = density (1=DD and 2=HD)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

10 - Return CompatiCard parameters

This function is used to return the parameters which

are currently set for a drive.  Since each drive gets

its own set of parameters, this function can return

different parameters according to the physical drive

address specified.

Entry:

AH = 10

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

AL = number of retries after I/O error (0-255)

CL = density (1=DD and 2=HD)

CH = double step flag (0=single step and  1=double

step)

BL = diskette drive type (0-7)

BH = step rate (in milliseconds)

ES is altered on exit

CY = 1 if error

AH = error code

11 - Return next sector ID

This function is used to return the next sector ID read

from the diskette.

Entry:

AH = 11

CH = cylinder (0-n)

CL = density (1=DD and 2=HD)

DH = head (0-1)

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

AL = sector size (0=128, 1=256, ...)

CL = sector number

CH = cylinder number

DH = side

CY = 1 if error

AH = error code

21 - Read DASD Type

This function is used to return change line

availability information.  The disk change line is not

supported on the CompatiCard II and is simulated using

the  system clock.  This function will always return  a

"no change line" for a CompatiCard II drive.

Entry:

AH = 21

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

AL = 0  drive not defined

1  no change line

CY = 1 if error

AH = error code

22 - Return change line status

This function is used to return change line

status information.  The disk change line is not

supported on the CompatiCard II and is simulated using

the system clock.  A "not active" value will be

returned if it has been less than 2 seconds since the

last diskette operation to the drive.

Entry:

AH = 22

DL = physical drive address (0-15)

Exit:

CY = 0 if AH = 0

CY = 1 if AH not 0

AH = 0  change signal not active

6  change signal active

23 - Set DASD type for format

This function is used to set some parameters before a

format operation. This function is provided for system

BIOS compatibility.  The use of function 9 is preferred

for new programs.

Entry:

AH = 23

AL = 1  360KB diskette in 360KB drive

2  360KB diskette in 1.2MB drive

3  1.2MB diskette in 1.2MB drive

4  720KB diskette in 720KB drive

DL = physical drive address (0-15)

Exit:

CY = 0 if no error

CY = 1 if error

AH = error code

24 - Set media type for format

This function is used to set some parameters before a

format operation. This function is provided for system

BIOS compatibility.  The use of function 9 is preferred

for new programs.

Entry:

AH = 24

CL = number of sectors per track

CH = number of cylinders

DL = physical drive address (0-15)

Exit:

ES:DI = pointer to 11 byte diskette drive parameter

table associated with the specified media

type

CY = 0 if error code is 0

CY = 1 if error code is not 0

AH = error code